# Annex No. 1

PROPOSAL

Under the EU European Agricultural Fund for Rural Development (EAFRD) Measure 4. 'Investments in tangible assets' Sub-measure 4.2 'Support for investments in processing'

| 1. Client: | AS "Ķekava Foods” |
| --- | --- |
| 2. Taxpayer registration number: | LV 50003007411 |
| 3. Client address: | Ziemeļu iela 55, Ķekava, Ķekavas nov., LV-2123 |
| 4. Ordered item and quantity: | Oven – 1 unit. |
| 5. Delivery location: | AS "Ķekava Foods”, Ziemeļu iela 55, Ķekava, Ķekavas nov., LV-2123 |
| 6. Delivery date: | 30.08.2026. |
| 7. Offer validity period: | till 30.08.2026. |
| 8. Technical specification: |  |

| Requirements |  | Supplier parameters |
| --- | --- | --- |
| Oven – 1 unit. |  | Manufacturer: Brand: Model: |
| 1 | Oven interior -3 transport wagons, 1050 x 1050 x 2100 mm (±100), 1-row configuration. |  |
| 2 | Loading and working weight, not less than, kg – 3 transport wagons x 400 kg. |  |
| 3 | Operable processes- showering, cleaning smoking, cooking, baking, Temperature range: up to 150 ° C. |  |
| 4 | Body made of stainless steel. |  |
| 5 | Minimum moisture protection class - IP65. |  |
| 6 | External dimensions of the oven: Depth 3460 ±40mm, Width: 1600mm ±50mm, Max height with fan, without grate 3650 ±50 mm. |  |
| 7 | Operable range; Room temperature up to +95 °C Relative humidity up to 99% RH. |  |
| 8 | Oven heating with electric heaters - built-in. |  |
| 9 | All nozzles (for washing system, water supply system), nozzle connections made of stainless steel - except flexible connections. |  |
| 10 | Number of programs not less than 20 pcs. |  |
| 11 | Smoke generator- designed for working with chipped wood (Alder, Beech) wood chips, grain size 8-16 mm. |  |
| 12 | Fan speed control from a frequency converter (the frequency converter is regulated from the control unit). |  |
| 13 | Opening the oven door from the left side. |  |
| 14 | Electrical cables located inside the oven body (power, control) are designed to operate at temperatures of +150 °C or higher. |  |
| 15 | The end of the flue (chimney) must be equipped with protection against various types of precipitation (rain, snow, hail, etc.). |  |
| 16 | The end of the furnace ventilation shafts (located outside the building - above the roof) must be equipped with protection against various types of precipitation (rain, snow, hail, etc.) while maintaining the intended flow rate. |  |
| 17 | Separate emulsion mixer for washing - chemical, water, etc. dispenser, mixing ratio from 0 to a minimum of 10% (chemical - alkali, acid). |  |
| 18 | Cooling evaporator integrated + 4°C. |  |
| 19 | Documentation - must include the equipment's operating instructions in Latvian, a spare parts catalog, an electrical diagram, and a pneumatic diagram. |  |
| 20 | Monitoring system with data retention for at least 4 months. |  |

Warranty Period: ______________

Defect Rectification: ____ hours.

Spare Parts Availability: ____ hours.

## Supplier Information

Company Name:

Registration No.:

Equipment Brand and Model:

Estimated Contract Execution Period:

Valid Until:

Price (EUR, excluding VAT):

Proposal Date:

Prepared by:

Position:

Email:

Phone:

Annex No. 2

AS "Ķekava Foods”

/Customer Name/

Declaration of Independent Proposal

Hereby, providing complete and truthful information,

____________________________________________________________________

[Supplier’s Name, Registration No.]

(hereinafter – Suppliert) with regard to the specific procurement procedure declares that:

- The Supplier has reviewed and agrees to the content of this declaration.

- The Supplier acknowledges its obligation to provide complete, comprehensive, and truthful information in this declaration.

- The Supplier’s procurement proposal has been signed by the Supplier’s authorized person(s).

- The Supplier confirms that the proposal has been submitted independently of competitors and without consultations, agreements, or arrangements. The Supplier has not communicated with any competitor regarding:

4.1. Prices;

4.2. Price calculation methods, factors (conditions), or formulas;

4.3. Intentions or decisions to participate or not participate in the procurement (to submit or not submit a proposal);

4.4. Submission of a proposal that does not meet the procurement requirements;

4.5. Quality, quantity, specifications, execution, delivery, or other conditions that must be addressed independently of competitors for the products or services covered by this procurement.

5. The Supplier has not knowingly, directly or indirectly, disclosed and will not disclose the terms of the proposal to any competitor before the official proposal opening date and time or the award of contract rights.

6. The Supplier acknowledges that the Competition Law establishes liability for prohibited agreements, providing for a fine of up to 10% of the offender’s net turnover from the last financial year, and the Supplier may be excluded from participating in the procurement procedure.

Date:

Signature: