Annex No. 2

TECHNICAL SPECIFICATION

PURCHASE OF WHEY PROCESSING EQUIPMENT

________________________

place, date

| Parameter | Requirement | THE PARTICIPANT shall provide information or proof against each requirement. |
| --- | --- | --- |
| Quantity | 1 pc. reverse osmosis unit with integrated polisher 1 pc. whey heating device 1 pc. whey vibration separator 1 pc. whey separation equipment integration 1 pc. whey concentrate cooling unit 1 pc. mechanical assembly materials for connecting units and storage 1 pc. MCC cabinets and cables 1 pc. Equipment and line automation 1 pc. Monitoring mechanical and electrical installation |  |
| Quality | Meets EU food industry equipment requirements and standards (in accordance with EU Regulation (EC) No. 1935/2004 on materials coming into contact with food) |  |
| Productivity line | RO unit with integrated polisher 30 m³/h, whey up to 30% dry matter 20 m³/h, skimmed milk up to 35% dry matter content 30 m³/h, whey vibration separator and whey heating |  |
| RO/PO unit | CIP dosing machine Dosing device for pH reduction |  |
| Whey heating | Whey: Heating: 72°C–75°C, Cooling 2–6°C Max. 2 K temperature difference between whey and hot water circulation |  |
| Retentate cooling | Whey and skimmed milk Cooling 2°C - 6°C, capacity 4,000 l/h - 5,500 l/h |  |
| Engines | Energy-reduced IE4 class motors, if applicable |  |
| Connection with: | Storage of raw whey Whey storage Whey/milk concentrate storage Polisher water storage Heat oscillation system |  |
| Mechanical assembly material | Valves (, pumps, measurement and control technology) Pipes and holders |  |
| Materials used | AISI316 stainless steel |  |
| Documentation | in Latvian or English (declarations of conformity, equipment passports, user manuals, maintenance, use and repair instructions, electrical and pneumatic diagrams (if applicable). |  |
| Labeling | CE |  |
| Compliance with European Mechanical Engineering Standards directives | Complies with EU directive (the declaration of conformity must be submitted to the Customer together with other documents specified in the specification, by signing the acceptance-handover protocol) |  |
| Security | Meets EU industry standard requirements |  |

________________________________

Name, surname, position

____________________

signature