Annex No. 2

TECHNICAL SPECIFICATION

PURCHASE OF WHEY SEPARATOR

________________________

place, date

| Parameter | Requirement | THE PARTICIPANT shall provide information or proof against each requirement. |
| --- | --- | --- |
| Quantity | 1 piece |  |
| Quality | Meets EU food industry equipment requirements and standards (in accordance with EU Regulation (EC) No. 1935/2004 on materials coming into contact with food) |  |
| Productivity | 30 m³/h |  |
| Product temperature | min. 30 °C max. 50 °C |  |
| Cream fat content | min. 10% max. 30% |  |
| Skimming efficiency | not more than 0.06% residual fat in skimmed milk whey |  |
| Installed engine power | Less than 40 kW |  |
| Materials used | AISI316 stainless steel |  |
| Solids collection container | 1 piece |  |
| Inductive flow meter | 2 pcs. |  |
| Pressure sensor | 1 piece |  |
| Control valves | 4 pcs. |  |
| Control system with monitoring | 1 piece |  |
| Documentation | in Latvian or English (declarations of conformity, equipment passports, user manuals, maintenance, use and repair instructions, electrical and pneumatic diagrams (if applicable). |  |
| Labeling | CE |  |
| Compliance with European Mechanical Engineering Standards directives | Complies with EU directive (the declaration of conformity must be submitted to the Customer together with other documents specified in the specification, by signing the acceptance-handover protocol) |  |
| Security | Meets EU industry standard requirements |  |

________________________________

Name, surname, position

____________________

signature