Annex No. 2

TECHNICAL SPECIFICATION

PURCHASE OF WHEY AND WATER TANKS

________________________

place, date

| Parameter | Requirement | THE PARTICIPANT shall provide information or proof against each requirement. |
| --- | --- | --- |
| Quantity | 2 pcs. tanks 100 m³ with mixer for cold whey (separated) 2 tanks for 100 m³ permeate 1 pc. tank with heat storage for 100 m³ water |  |
| Quality | Meets EU food industry equipment requirements and standards (in accordance with EU Regulation (EC) No. 1935/2004 on materials coming into contact with food) |  |
| Tank model | vertical |  |
| Design | Temperature 5°C–95°C Atmospheric pressure |  |
| Metric | Inner diameter max. 3500 mm Outer diameter 3640 mm Spout height 1000 mm |  |
| Materials used | AISI316 – in contact with the product AISI316 or AISI304 – not in contact with the product |  |
| Insulation | Whey and permeate tank Cylinder and upper 100 mm mineral wool, lower 80 mm PU Water tank Cylinder, upper and lower 150 mm mineral wool |  |
| Tanks (general) | upper lower part: 1 x ventilation hood with insect screen and steam pipe 2 x transport loops 1 x cleaning nozzle with spray ball, removable the pipeline has been demolished 1 x sleeve for complete communication (included) cylinder shell: 1 x nameplate 1 x mixer flange 1 x Mixer cleaner Lower floor: 1 x pipe nipple DN 100 DIN 11850-2 with flow interrupter, welding end 1 x sleeve for detecting empty elements (included) 1 x DRD flange 1 x immersion sleeve PT 100 (included) 1 x oval hatch 410 x 510 mm with O-ring seal and Border switch |  |
| General | 1 x roof edge railing for each tank 1 x test statics part for each tank Each tank includes 1 design revision |  |
| Appendix | Either with ground anchors or concrete foundations |  |
| Additions | 1x vertical ladder used for all tanks Handrails on top of tanks Non-slip diamond pattern on the top of the tanks 1x on-site installation |  |
| Heat storage tank | 1 x pipe end DN 125 with flange, inside with 90° bend to the wall (inlet), pipe pulled out to approx. 2000 mm 1 x pipe end DN 100 with flange, inside with 90° bend to the pipeline has been laid to approximately 6000 mm 1 x pipe nozzle DN 125 with flow interrupter, welding end 6 x immersion sleeves PT 100 (included) 6 x socket level detectors (included) |  |
| Documentation | in Latvian or English (declarations of conformity, equipment passports, user manuals, maintenance, use and repair instructions, electrical and pneumatic diagrams (if applicable). |  |
| Labeling | CE |  |
| Compliance with European Mechanical Engineering Standards directives | Complies with EU directive (the declaration of conformity must be submitted to the Customer together with other documents specified in the specification, by signing the acceptance-handover protocol) |  |
| Security | Meets EU industry standard requirements |  |

________________________________

Name, surname, position

____________________

signature