## Annex No. 2

## Technical Specification

### Purchase of Industrial High-Capacity Controlled Defrosters

| Parameter | Requirement |
| --- | --- |
| THE PARTICIPANT shall provide information or proof against each requirement. |  |
| Quantity | 2 pcs. – Industrial Defrosting Cabinets. |
| Purpose | Controlled, rapid, and consistent thawing of high-value food products (e.g., fish, shellfish, meat, vegetables) while maximizing yield and minimizing waste. Suitable for large batch processing. |
| Capacity & Operational |  |
| * Nominal Load Capacity | 500 kg – 510 kg (per cycle). |
| * Airflow System | Unique Horizontal Reversing Airflow System (or equivalent technology) to ensure rapid and consistent heat and humidity transfer across the entire load. |
| * Temperature Control | Microprocessor control (e.g., Siemens S7 or equivalent industrial system) with unique temperature curve monitoring of core, surface, and chamber temperatures. |
| * Programs | Must allow for individual temperature curves for each product type. |
| * Humidity System | Must include a Humidity System (water spray system) and an automatic sanitizing system to significantly reduce water and energy costs. |
| * Post-Cycle Function | Must include a Refrigeration option to hold the product at a set point temperature or provide pull-down refrigeration to maintain critical surface temperatures after the defrosting cycle. |
| External Dimensions (Single Unit) |  |
| * Doors | 1 door (single body unit). |
| * Overall Length (A) | 1660 mm – 1680 mm. |
| * Overall Width (B) | 1960 mm – 1980 mm. |
| * Overall Height (C) | 2160 mm – 2180 mm. |
| * Max. Depth (D) (Fan and Control Box not included) | 2500 mm – 2520 mm. |
| Site Requirements & Connections |  |
| * Electrical Supply | 380V – 440V, 3 phase and neutral, 50 Hz |
| * Cooling Power (Nominal) | Cooling Power Output: 40 kW – 42 kW. |
| * Heating Power (Nominal) | Heating Power Output: 25 kW – 26 kW. |
| * Water Supply | Dedicated drinkable water supply required for the water spray system: 40 litres/hour – 45 litres/hour. |
| * Installation | Requires a flat, level concrete floor and internal cement seal. Must have drainage to outside the unit. |
| * Noise Level | Noise level at the operator position: less than 85 dB(A). |
| Complete Functional Set (Mandatory) | The Supplier must fully complete the equipment, understanding the intended purpose (controlled defrosting). If further information is required, the Supplier must contact the procurement officer/person responsible for the purchase. The offer must explicitly include: |
|  | • The main defroster cabinet with integrated fan and control system. |
|  | • All necessary components for the automatic control, temperature/humidity sensors, refrigeration, and water spray systems. |
|  | • Installation materials and final utility connections. |
|  | • Commissioning and comprehensive staff training. |
| Documentation | To be supplied in Latvian or English: user manual, electric diagram, maintenance instructions, and CE Declaration of Conformity. |
| Guarantee | Minimum 24 months full manufacturer or supplier warranty. |