Annex No. 2

TECHNICAL SPECIFICATION

PURCHASE OF INDUSTRIAL MOBILE OZONE AIR GENERATORS

place, date

The following table:

| Parameter | Requirement |
| --- | --- |
| THE PARTICIPANT shall provide information or proof against each requirement. |  |
| Quantity | 10 pcs. – Mobile industrial ozone generator designed for air sanitization. |
| Brand, model offered | Brand name (for identification purposes) and model name (for identification purposes). |
| Purpose | Disinfection, sanitization, and deodorization of large indoor areas (e.g., cold rooms, production halls, commercial buildings) by eliminating moulds, bacteria, viruses, and bad smells. |
| Area Capacity | • Maximum Effective Area: Capable of sanitizing areas up to 1400 – 1500 sq.m. in a single cycle. |
| Ozone Output | • Production Rate: Rated Ozone generation between 100 – 120 g/h (grams per hour). |
| Construction & Materials | • Material: Entire construction (housing, base) must be made of Stainless Steel AISI 304 or equivalent high-quality material. • Design: Floor-standing tower/column unit for optimal air circulation. |
| Air Circulation | • Fan System: Equipped with a powerful dual-fan system (or equivalent) to ensure rapid and effective dispersion of ozone throughout the treated area. |
| Control System | • Timer: Integrated timer/control system with quick-set functionality (e.g., NFC controlled timer or dedicated app control) for fast initiation of the sanitizing cycle. • Adjustability: Digital or electronic adjustment of the cycle duration (0 – 24 hours). |
| Power & Mobility | • Power Consumption: 700 W – 800 W. • Voltage: 230V / 1-phase / 50 Hz. • Mobility: Mounted on a stable base or fitted with wheels/castors for easy transportation between rooms. |
| Dimensions (Approximate) | • Width: 300 – 350 mm. • Depth: 320 – 350 mm. • Height: 1750 – 1850 mm |
| Complete Functional Set (Mandatory) | Supplier Responsibility: The supplier is fully responsible for providing a complete and operational unit ready for immediate use. The offer must include: • Ozone generation tower/unit. • Integrated fan system. • Control panel/timer. • Power cable. • Safety signs and user guidelines (for safe handling of ozone). |
| Documentation | To be supplied in Latvian or English: user manual, technical specifications, maintenance instructions, CE Declaration of Conformity. |
| Compliance | Equipment must comply with EU safety regulations and all relevant standards for biocidal/air treatment equipment. CE marking mandatory. |
| Guarantee | Minimum 24 months full manufacturer or supplier warranty. |

Name, surname, position

signature