## Annex No. 2

## Technical Specification

### Purchase of Industrial Modular Combi Oven and Static Deck Baking Complex

| Parameter | Requirement |
| --- | --- |
| THE PARTICIPANT shall provide information or proof against each requirement. |  |
| Quantity | 1 pc. – Industrial Modular Combi Oven Complex, combining convection and static deck baking units. |
| Purpose | Provides maximum versatility and production capacity for various confectionery, bakery, and pizza products. The complex must offer an excellent balance between production quantity and baking quality |
| Configuration | The unit must consist of the following stacked and electrically connected functional modules: |
|  | 1. Convection Oven Chamber (Upper): Ventilated chamber capable of holding 6 trays (40x60 cm). |
|  | 2. Static Oven Chamber (Middle): Deck oven module with independent control, capable of holding 2 trays (40x60 cm). |
|  | 3. Static Oven Chamber (Lower): Deck oven module with independent control, capable of holding 2 trays (40x60 cm). |
|  | 4. Base Module: Standard base frame/support structure (BM), suitable for housing ancillary equipment or providing stable support. |
| External Dimensions (Total Complex) |  |
| * Total External Width | 960 mm – 980 mm. |
| * Total External Depth | 1200 mm – 1220 mm. |
| * Total External Height | 2100 mm – 2200 mm . |
| Energy & Heating |  |
| * Heating Source | Electric. |
| * Total Connected Load | 23.0 kW – 24.5 kW. |
| * Voltage / Frequency | 400V / 3N / 50 Hz or 60 Hz. |
| Convection Chamber (Upper) |  |
| * Capacity | 6 trays (40x60 cm). |
| * Max. Temperature | 250C – 270C (Convection). |
| * Features | Standard internal vaporizer (steamer). Door: Double glazed in tempered glass. |
| * Control | Touch screen control panel. Must include core probe and washing function. |
| Static Deck Chambers (Middle/Lower) |  |
| * Capacity (Each) | 2 trays (40x60 cm). |
| * Max. Temperature | 380C – 400C (Static Deck). |
| * Features | Doors: Double glazed ceramic glass. Floors: Option for refractory ceramic or embossed metal sheet cooking surface. |
| * Control (Each Module) | Digital control panel with 20 program capacity21. |
| Ventilation & Hood (Mandatory Options) |  |
| * Hood Type | Must include a Suction Hood (hotte aspirante) (mandatory option). |
| * Air Volume | Suction Capacity: 400 m3/h – 600 m3/h (or higher). |
| * Hood Power (if motorized) | Max. Power Consumption: 0.20 kW – 0.30 kW. |
| Mobility & Safety |  |
| * Base | Base module (BM) must be equipped with lockable swivel wheels or adjustable feet. |
| Complete Functional Set (Mandatory) | The Supplier must ensure the delivery of a fully integrated and functional oven complex, including all inter-module connections, the specified suction hood with motor, and all control systems. The offer must explicitly include: installation materials, commissioning, and comprehensive staff training. |
| Documentation | To be supplied in Latvian or English: user manual, electric diagram, maintenance instructions, and CE Declaration of Conformity. |
| Guarantee | Minimum 24 months full manufacturer or supplier warranty. |