## Annex No. 2

## Technical Specification

### Purchase of Custom Industrial Caviar Washing and Sieving Module (Post-Separator Stage)

| Parameter | Requirement |
| --- | --- |
| THE PARTICIPANT shall provide information or proof against each requirement. |  |
| Quantity | 1 pc. – Custom-Designed Caviar Washing & Sieving Module (Post-Separator Stage). |
| Intended Use | Gentle washing, controlled rinsing, fine sieving, and quick draining of delicate fish roe directly after membrane separation (prior to salting/marination). |
| 1. Capacity and Performance |  |
| * Hourly Throughput | 15 kg/h – 25 kg/h. |
| * Monthly Throughput | Up to 4,000 kg/month. |
| * Damage Rate (Critical KPI) | Mechanical damage to roe must be held to less than 1% of the product mass. |
| * Rinsing Time per Batch | 20 seconds – 60 seconds. |
| 2. Washing Tank (Primary Module) |  |
| * Tank Material (Critical) | AISI 316 Stainless Steel (Mandatory for all parts in contact with product, brine, or water). |
| * Useful Volume | 70 L – 120 L. |
| * Internal Dimensions (L x W x H) | 800 mm – 900 mm (L) x 600 mm – 650 mm (W) x 380 mm – 420 mm (H). |
| * Bottom Design | Fully sloped toward the drain (minimum 2% slope). |
| * Internal Finish | Rounded corners > 10; fully welded with no dead zones; Surface roughness < 0.8. |
| 3. Drainage and Plumbing |  |
| * Drain Outlet (Primary) | DN40 or DN50 hygienic valve (e.g., tri-clamp or butterfly sanitary valve) for instant water disposal. |
| * Drainage Channel | Integrated channel placed beneath the perforated insert, continuously sloped (min. 1.5% fall). |
| * Rinsing Water System | Spray Bar Module (AISI 316) providing a gentle, uniform water curtain over the roe surface. |
| * Water Supply | Connection to cold fresh water line; flow must be regulated (optional pressure regulator 0.5 - 2. |
| 4. Sieving Module (Perforated Insert) |  |
| * Insert Material | AISI 316 Stainless Steel. |
| * Perforation Size (Critical) | Primary sieve layer perforation must be 2 mm – 3 mm. |
| * Format | Removable, custom-fit insert (or GN 2/1 standard perforated tray equivalent). |
| * Function | Allows the removal of micro-residues and quick draining while retaining roe. |
| 5. Frame and Mobility |  |
| * Frame Material (Preferred) | AISI 316 Stainless Steel (Hygienic round-tube frame 40 – 50 mm). |
| * Total Height (Working) | Ergonomic manual handling height: 850 mm – 900 mm. |
| * Mobility | Must be available on a stainless steel table base or 4 stainless swivel wheels (2 with brakes). |
| * Floor Clearance | Leg construction must allow full floor cleaning under the table. |
| * Levelling | Adjustable anti-slip levelling feet (minimum 30 mm adjustment). |
| 6. Optional/Required Accessories |  |
| * Product Handling | Stainless steel mixing/perforated paddle suitable for manual agitation. |
| * Protection | Side splash guards against water/aerosol contamination. |
| * Waste Collection | Compatible GN 1/1 stainless waste container mounted under the drain (optional comfort setup). |
| 7. Compliance and Safety |  |
| * Hygienic Design | Designed according to HACCP, ISO 22000, and EHEDG hygienic design principles. |
| * Sanitation | Must tolerate high-pressure washing, foam cleaning, and daily sanitation chemicals. |
| * Mandatory Standards | Must comply with: EN 1672-2 and EU Food Contact Material Regulation EC 1935/2004. |
| * Marking | CE marking is mandatory. |
| Complete Functional Set (Mandatory) | The Supplier must deliver a single fully functional, custom-configured module ready for post-separator processing, including the main tank, spray bar, perforated insert, drain components, frame, and paddle. The offer must explicitly include: the final system configuration, installation materials, commissioning, and comprehensive staff training. |
| Guarantee | Minimum 24 months full manufacturer or supplier warranty. |