## Annex No. 2

## Technical Specification

### Purchase of Custom Industrial Gentle Caviar Mixing System (Stirring Frame Type)

| Parameter | Requirement |
| --- | --- |
| THE PARTICIPANT shall provide information or proof against each requirement. |  |
| Quantity | 1 pc. – Industrial Gentle Caviar Mixing System (Item Code: 133B.5 – Stirring Frame Concept or Equivalent). |
| Purpose | To ensure gentle mixing of fish roe/caviar with salt, preservatives, and other recipe ingredients, preventing damage to the roe structure and ensuring uniform distribution within a short cycle time. |
| 1. Operational Capacity |  |
| * Monthly Throughput | System must support a production capacity of up to 4,000 kg/month (based on one 8-hour shift). |
| * Batch Size | Designed to handle one batch of 20 kg – 50 kg of roe/caviar. |
| * Cycle Time | Typical mixing cycle duration must be kept short: 1 minute – 3 minutes per batch. |
| * Daily Capacity | Expected daily throughput (8h shift): 200 kg – 300 kg. |
| 2. Technological Concept (Mandatory) |  |
| * Principle | Must be based on the "Rotating Tub / Stirring Frame" principle, where the unit rotates the specialized container rather than mixing the contents internally with paddles. |
| * Mixing Action | Mixing must occur through a gentle tumbling / rocking action, minimizing shear forces and avoiding direct mechanical contact with the roe mass. This is a crucial requirement for maintaining roe integrity. |
| 3. Stirring Frame (Main Unit) |  |
| * Frame Construction | Robust frame made of Stainless Steel (AISI 304 minimum), designed to securely hold and rotate the caviar tubs. |
| * Material (Contact Zones) | AISI 316 Stainless Steel is preferred for all zones susceptible to contact with brine/salt (external splash areas). |
| * Rotation Speed | Motorized rotation must feature variable speed control (low RPM), with an orientational operating range of 5 RPM – 25 RPM. |
| * Rotation Direction | Rotation capability must include single direction or reversible direction (reversible is preferred). |
| * Control | Control panel mounted on the frame with ON/OFF, speed control, and emergency stop functionality. |
| * Tub Fixing | Tub must be securely fixed onto the frame using mechanical clamps or an equivalent press/locking system. |
| * Safety & Hygiene | Unit must be wash-down ready (IP55 or higher electrical rating). Must include vibration and noise dampening. Compliance with EN 1672-2 (food machinery hygiene design) is mandatory. |
| 4. Caviar Tubs (Mixing Containers - Included) |  |
| * Quantity Included | 1 pc. – 2 pcs. Stainless Steel Tubs (vessels) specifically designed for mounting on the stirring frame. |
| * Material (Critical) | Tub material must be AISI 316 Stainless Steel (mandatory due to direct contact with brine/salt). |
| * Capacity (Volume) | Nominal tub capacity: 40 L – 80 L. |
| * Design | Shape must be cylindrical or gently conical, with no sharp edges. Features exterior handles for easy transfer. |
| 5. Safety, Ergonomics & Integration |  |
| * Safety Features | Includes safety mechanisms (e.g., protective grating or mechanical brake) compliant with EU safety logic. |
| * Ergonomics | Tub must be easily placed onto and removed from the frame by one operator. |
| * Mounting | Design must be easily integrated into the salting area. Construction on wheels with brakes is preferred (optional feature). |
| * Space/Handling | Must include a designated space/method for resting the tub when not in use. |
| Complete Functional Set (Mandatory) | The Supplier must fully complete the equipment, understanding the intended purpose (gentle caviar/roe mixing) and ensuring the inclusion of all required components and accessories for operation, including the 1-2 specialized AISI 316 tubs. |
| Documentation | Must include: Technical Data Sheet, Materials Specification, Operating Instructions, Safety Instructions, and CE Declaration of Conformity. |
| Guarantee | Minimum 24 months full manufacturer or supplier warranty. |