## Annex No. 2

## Technical Specification

### Purchase of Industrial Brine Mixing and Preparation Units

| Parameter | Requirement |
| --- | --- |
| THE PARTICIPANT shall provide information or proof against each requirement. |  |
| Quantity | 2 pcs. – Industrial Brine Mixing and Preparation Units. |
| Purpose | Efficient, standardized, and rapid preparation of any type of liquid brine, marinade, or sauce by mixing liquids and powdered additives. Must be capable of efficiently blending solids like flaked ice. |
| Capacity & Performance |  |
| * Container Volume | 290 L – 310 L. |
| * Mixing Principle | Must utilize a suction flow principle (or equivalent turbo-mixing mechanism) for rapid and homogeneous dissolution of solids and liquids. |
| * Mixing Capability | Must include an agitator or equivalent mixing mechanism capable of processing flaked ice. |
| * Output System | Brine conveying/output via a powerful stainless steel centrifugal pump. |
| Construction & Hygiene |  |
| * Material | Hygienic, foodstuff-compliant structure made entirely of stainless steel. |
| * Design | Easy-care design with a cylindrical tank and rounded bottom allowing for complete emptying. |
| * Monitoring | Must include a highly-visible filling level display. |
| * Cleaning | Designed for easy cleaning. |
| External Dimensions |  |
| * Length | 1180 mm – 1220 mm. |
| * Width | 880 mm – 920 mm. |
| * Height | 1830 mm – 1870 mm. |
| Technical & Energy |  |
| * Connected Value (Power) | 1.3 kW – 1.6 kW. |
| * Electrical Supply | 400 V, 3 Phase + Neutral + Earth |
| Optional Features (Required if Available and must be quoted) |  |
| * Weighing | Must be compatible with weighing cells for precise batch monitoring/recording. |
| * Cooling | Must offer an integrated cooling jacket (or be ready for connection to external cooling devices) for temperature control. |
| * Control System | Must be compatible with an advanced touchscreen control unit and recipe management system. |
| * Additional Outputs | Must support adding additional outputs/valves for connection to multiple consumers. |
| Complete Functional Set (Mandatory) | The Supplier must ensure the delivery of two fully functional units, complete for brine mixing/delivery, including the powerful centrifugal pump and agitator. The Supplier must ensure that the equipment, including all necessary integrated components and required connections, fully performs its assigned tasks. The offer must explicitly include: installation materials, commissioning, and comprehensive staff training. |
| Documentation | To be supplied in Latvian or English: user manual, electric diagram, maintenance instructions, and CE Declaration of Conformity. |
| Guarantee | Minimum 24 months full manufacturer or supplier warranty. |