## Annex No. 2

## Technical Specification

### Purchase of Industrial Salting / Marination Table

| Parameter | Requirement |
| --- | --- |
| THE PARTICIPANT shall provide information or proof against each requirement. |  |
| Quantity | 5 pc. – Industrial Salting / Marination Table with Perforated Top and Drainage. |
| Intended Use | Manual salting, wet/dry marination, draining, and pre-processing of fish in a low-risk food-processing zone. |
| 1. Dimensions (Tolerance: ±10 mm) |  |
| * Length (L) | 1500 mm. |
| * Width (W) | 800 mm. |
| * Height (H) | 900 mm. |
| 2. Construction Requirements |  |
| * Material (Critical) | All parts in contact with product, water, brine, or tools must be made from AISI 316 stainless steel. |
| * Structural Frame | May be AISI 304 or 316; AISI 316 is preferred for full corrosion resistance. |
| * Welding & Finish | Welding must be continuous, polished, with no sharp edges, dead zones, or crevices (hygienic design). |
| 3. Worktop Design |  |
| * Worktop Material/Thickness | Perforated worktop, AISI 316, thickness equal to or greater than 2.0 mm. |
| * Perforation Pattern | Round holes 5 – 8 mm, evenly distributed, or equivalent hygiene-approved pattern. |
| * Access | Worktop must be fully removable for washing or securely mounted on hinges for lifting. |
| * Corrosion Resistance | Must be chemically designed to withstand wet brining, salt exposure, and acidic marinades. |
| * Load Capacity | Reinforced worktop load capacity must be equal to or greater than 80 kg evenly distributed. |
| 4. Drainage System |  |
| * Main Feature | Integrated drainage channel/gutter placed directly under the perforated top. |
| * Slope | Channel must be sloped (minimum 1.5% fall) toward a drain outlet. |
| * Drain Outlet | Drain outlet diameter: 40 – 50 mm (for connection to floor drain or GN container). |
| * Grid | Includes a removable drainage grid made of AISI 316 (full-length grid or two-section grid). |
| * Cleanability | System must allow easy removal and cleaning with no hidden zones. |
| 5. Frame & Legs |  |
| * Frame Type | Hygienic round-tube frame |
| * Floor Clearance | Leg construction must allow full floor cleaning under the table. |
| * Adjustability | Adjustable anti-slip levelling feet . |
| * Splashback (Optional) | Optional splashback on 1–3 sides (height 50 mm – 100 mm). |
| 6. Optional Features (Required if Available) |  |
| * Waste Container | Compatible GN 1/1 stainless waste container mounted under the drain. |
| * Undershelf | Undershelf (AISI 304 or 316) – must be open, easy to wash, and removable. |
| * Side Protection | Side splash guards against water/aerosol contamination. |
| 7. Hygiene & Safety |  |
| * Design Principles | Designed according to HACCP, ISO 22000, and EHEDG hygienic design principles. |
| * Sanitation | Must tolerate high-pressure washing, foam cleaning, and daily sanitation chemicals. |
| * Surface Finish | All surfaces must be smooth, polished, with roughness average < 0.8 μm. |
| 8. Compliance |  |
| * Standards | The table must comply with: EN 1672-2 (Food Processing Machinery – Hygiene Requirements) and EU Food Contact Material Regulation EC 1935/2004. |
| * Marking | CE marking is mandatory. |
| Complete Functional Set (Mandatory) | The Supplier must ensure the delivery of a single fully functional, industrial-grade Salting/Marination Table with all specified materials, construction features, and optional accessories (if commercially available). The offer must explicitly include: the table structure, perforated top, drainage grid, waste outlet, levelling feet, and the CE Declaration of Conformity. |
| Guarantee | Minimum 24 months full manufacturer or supplier warranty. |