## Annex No. 2

## Technical Specification

### Purchase of Industrial Round Brine Storage Tanks with Lid (Fish Brine Use)

| Parameter | Requirement |
| --- | --- |
| THE PARTICIPANT shall provide information or proof against each requirement. |  |
| Quantity | 4 pcs. – Industrial Vertical Cylindrical Storage Tanks with Lid. |
| Purpose | Designed for the standardized storage and preparation of high-concentration salt solutions (Brine), marinades, and other liquids used in the food industry, including fish processing and water purification systems. |
| 1. Capacity and Dimensions |  |
| * Nominal Volume | 150 Litres – 165 Litres. |
| * External Diameter | 590 mm – 610 mm. |
| * External Height (Max.) | 700 mm – 750 mm. |
| * Nominal Weight (Empty) | 10 kg – 15 kg. |
| 2. Construction & Material |  |
| * Material | Manufactured from UV-stabilized, medium-density polyethylene (MDPE). |
| * Construction | Seamless, one-piece construction via rotational moulding for maximum strength. |
| * Hygiene | Internal and external surfaces must be smooth for easy cleaning. |
| * Lid | Supplied with a secure screw-down lid (min. diameter 355 mm). |
| 3. Brine / Salt Compatibility (Mandatory) |  |
| * Concentration Tolerance | Suitable for storing high-concentration salt solutions ranging from 3.5% up to 26% NaCl (saturated solution), applicable for brining foods. |
| * Salt Grid (Required if Available) | Must be compatible with optional salt grids for use with pellet or granular salt. |
| * Auxiliary Use | Tank design must be suitable for use as an auxiliary agent in water softening or purification systems where regeneration involves brine (6–12% NaCl). |
| 4. Compliance & Guarantee |  |
| * Use Approval | Tank materials must be approved as suitable for use in the Food Industry. |
| * Standard Outlet | Must include a primary outlet fitting (minimum 1.5-inch BSP Male plastic outlet). |
| Complete Functional Set (Mandatory) | The Supplier must ensure the delivery of four fully functional storage tanks, ready for immediate deployment. The offer must explicitly include the tanks, lids, and standard outlet fittings. |
| Documentation | To be supplied in Latvian or English: user manual, maintenance instructions, and CE Declaration of Conformity. |
| Guarantee | Minimum 24 months full manufacturer or supplier warranty. |