## Annex No. 2

## Technical Specification

### Purchase of Industrial ATEX-Certified Glueboard Flykillers

| Parameter | Requirement |
| --- | --- |
| THE PARTICIPANT shall provide information or proof against each requirement. |  |
| Quantity | 2 pcs. – Industrial ATEX-Certified Glueboard Flykillers. |
| Purpose | Hygienic control of flying insects in sensitive areas with a medium to high explosion risk (e.g., food manufacturing, distilleries, chemical processing). The unit must capture insects using glueboards, adhering to strict explosion protection standards. |
| 1. ATEX Certification (CRITICAL) |  |
| * Certification Level | Certified to ATEX category 2. |
| * Zone Rating | Suitable for use in Zone 1 & 2 (gas environment) / Zone 21 & 22 (dust environment). |
| 2. Construction and Durability |  |
| * Construction Material | AISI 316 stainless steel grade frame for maximum corrosion resistance. |
| * Casing | IK08 graded and toughened glass casing. |
| * UV Tubes | Must use FEP coated shatterproof UV tubes. |
| 3. Dimensions and Performance |  |
| * External Height (H) | 830 mm – 840 mm |
| * External Width (W) | 600 mm – 610 mm |
| * External Depth (D) | 210 mm – 220 mm |
| * Coverage Area | Suggested area coverage: 115 m2 – 125 m2 |
| * Weight | 11.0 kg – 11.5 kg. |
| 4. Technical and Maintenance |  |
| * Electrical | Equipped with a multi-voltage electronic ballast. |
| * Servicing | Features a detachable light fitting for easy installation and servicing. |
| * Hygiene Access | Includes a debris aperture incorporating glueboard runners for easy cleaning. |
| 5. Complete Functional Set (Mandatory) | The Supplier must ensure that all components and preparation of the machine are completed for full operation. If data is lacking, the supplier must contact the procurement department. The offer must explicitly include: |
|  | • The main unit with ATEX certification. |
|  | • Shatterproof UV tubes for each unit. |
|  | • A set of starter glueboards for immediate operation. |
|  | • Installation materials and final utility connections. |
|  | • Commissioning and comprehensive staff training. |
| Documentation | To be supplied in Latvian or English: user manual, electric diagram, maintenance instructions, and CE Declaration of Conformity. |
| Guarantee | Minimum 24 months full manufacturer or supplier warranty. |