## Annex No. 2

## Technical Specification

### Purchase of Industrial Smoke Cooking Trolleys

| Parameter | Requirement |
| --- | --- |
| THE PARTICIPANT shall provide information or proof against each requirement. |  |
| Quantity | 4 pcs. – Industrial Smoke Cooking Trolleys. |
| Purpose | Designed for high-temperature and heavy-load product processing (meat, poultry, fish, baked goods) in smoking chambers, thermal ovens, or washing machines. Must ensure maximum stability and resistance to high temperatures and aggressive cleaning agents. |
| 1. Dimensions and Capacity (Based on RKW-VM 10E class) |  |
| * Maximum Total Load | Min. 700 kg – Max. 800 kg. |
| * Layers/Levels | Must be designed with a minimum of 17 layers/levels (grids) for hanging or lying product processing. |
| * External Height (H) | 1980 mm – 2050 mm. |
| * External Width (W) | 1000 mm – 1050 mm. |
| * External Depth (D) | 800 mm – 850 mm. |
| 2. Construction and Materials |  |
| * Material | Made of Stainless Steel 1.4301 (AISI 304). |
| * Frame Construction | Stable frame construction (e.g., 40 mm profile). |
| * Welding | Requires neat welding and first class stained design (clean, pore-free welding). |
| * Grids/Shelves | 17 welded round tube grids (10 mm). Distance between layers: 330 mm. |
| * Design | Special M-shape design (RKW-VM Type) or an equivalent design that optimizes the utilization of oven/smoker space. |
| 3. Mobility and Transport |  |
| * Castors | Must be equipped with 6 polyamide fixed castors (150 mm). |
| * Temperature Resistance | Polyamide castors must be temperature resistant up to 130C. |
| * Wheel Configuration | The middle pair of wheels must be increased to facilitate trolley turning (tilting moment). |
| 4. Compliance and Completion |  |
| * Marking | Frame construction above must include a TARA badge on both sides. |
| Complete Functional Set (Mandatory) | The Supplier must ensure that all components and preparation of the trolley are completed for full operation (including suitability for smoking/cooking). If data is lacking, the supplier must contact the procurement department. The offer must explicitly include: |
|  | • The complete trolley structure with all 17 fixed layers/grids. |
|  | • All 6 high-temperature resistant castors. |
|  | • Installation materials. |
|  | • Documentation (user manual, maintenance instructions) and CE Declaration of Conformity. |
| Guarantee | Minimum 24 months full manufacturer or supplier warranty. |