## Annex No. 2

## Technical Specification

### Purchase of Industrial Vertical Autoclaves with Pressure Support

| Parameter | Requirement |
| --- | --- |
| THE PARTICIPANT shall provide information or proof against each requirement. |  |
| Quantity | 2 pcs. – Industrial Vertical Autoclaves with Pressure Support. |
| Intended Use (Purpose - Critical) | The system is exclusively designed for the cooking, sterilization, and pasteurization of food in containers using a temperature probe. The primary function is to pasteurize roe/caviar in various sealed containers while preserving organoleptic properties and nutritional characteristics. |
| * Packaging Compatibility (Mandatory) | Must be fully compatible with, and designed to process, Glass, Metal, and Plastic containers of various sizes, specifically those intended for roe/caviar packaging: |
|  | • Glass: 50 g, 100 g, 200 g, 250 g, 500 g. |
|  | • Metal: 30 g, 50 g, 100 g, 125 g, 200 g, 250 g, 500 g. |
|  | • Plastic: 100 g, 200 g, 250 g, 500 g. |
| 1. Technical Specifications (CFS-75V Class) |  |
| * Nominal Capacity (Vessel Volume) | 65 Litres – 85 Litres. |
| * Maximum Working Temperature | 120C (Required for sterilization and pasteurization). |
| * Maximum Working Pressure | Must operate with pressure support to avoid deforming or breaking sensitive containers (e.g., plastic or thin metal). |
| * Heating | Electric heating. |
| * Cooling | Must include an integrated Fast Cooling system (water shower) to avoid overcooking the packaged product. |
| 2. Control and Performance |  |
| * Process Control | Precise control of the entire process using a Heart Temperature Probe placed on a product sample. |
| * Traceability | Must allow time optimization and perfect control via F0 value (sterilization) and P0 value (pasteurization).. |
| * Memory | Storage capacity for up to 50 programs. |
| * Data Logging | Must include integrated functions for data management, monitoring, and traceability of each process. |
| 3. Construction and Requirements |  |
| * Material | Pressure vessel and critical components must be made of Stainless Steel (AISI 304 or 316). |
| * Height (H) | External Height: 1250 mm – 1650 mm (Range covering common vertical models). |
| * Power Supply | Standard three-phase industrial voltage (e.g., 400V, 50/60 Hz). |
| 4. Mandatory Accessories and Complementary Systems |  |
| * Lifting System | Mandatory inclusion of an integrated basket lifting crane (or mobile lifting device - MOB-LIFT equivalent) for ergonomic loading/unloading of baskets. |
| * Baskets and Separators | A complete set of standard and custom baskets (CVT/CV/BCVT), specialized basket frames (KSEP), and silicone separators (SILIC-SEP) for handling different container types. |
| * Probes and Extensions | Must include a minimum of 2 additional flexible core temperature probes (PT-2-CFS-H equivalent) and a probe extraction tool (EXT-TP). |
| * Control Software and Data | Mandatory inclusion of the specialized Software Package (SW8000 equivalent) for advanced control, plus the necessary hardware for data download (e.g., DLDISK3618 equivalent). |
| * Control and Safety | Must include the specialized Pressure Sensor and Control system (PRSENSACLAV equivalent) for precise pressure support. |
| Complete Functional Set (Mandatory) | The Supplier must ensure the delivery of two fully operational autoclaves, including all specified mandatory accessories and software for roe/caviar processing, fully meeting the specified capacity and compatibility. |
| Documentation | To be supplied in Latvian or English: user manual, technical data sheet, and CE Declaration of Conformity. |
| Guarantee | Minimum 24 months full manufacturer or supplier warranty. |