Annex No. 2

TECHNICAL SPECIFICATION

PURCHASE OF AUTOMATIC PLANETARY MIXER

place, date

The following table:

| Parameter | Requirement |
| --- | --- |
| THE PARTICIPANT shall provide information or proof against each requirement. |  |
| Quantity | 2 pcs. – Heavy-duty automatic planetary mixer with complete accessory set. |
| Brand, model offered | Brand name (for identification purposes) and model name (for identification purposes). |
| Purpose | Professional machine for emulsions, mixtures, and dough in pastry and gastronomy. Capable of intensive use with total tool disengagement for easy bowl removal without removing the tool. |
| Bowl Capacity | • Volume: 70 – 80 litres (Target: 75 litres). |
| Performance & Speed | • Speed Control: Electronic variator with inverter technology . • Speed Levels: Minimum 5 speeds + pause function + continuous variation . • Transmission: Belt variator ensuring high torque at low speeds. |
| Motor & Power | • Power: 2.5 kW – 3.5 kW (Target: 3.0 kW) . • Voltage: 400V / 3 ph / 50 Hz. |
| Construction | • Structure: Heavy-duty steel sheet structure of high thickness. • Gears: Hardened, tempered, and rectified gears in an oil bath . • Finish: Painted with certified food-safe paint; critical parts treated with nickel plating for food use . • Protection Grid: Easily removable/turnable for cleaning. |
| Bowl Lifting System | • Type: Motorized automatic bowl lifting and lowering. • Stroke: Long bowl stroke to ensure total tool disengagement, allowing the bowl to be removed without taking off the tool. |
| Control Panel | • Type: Programmable digital control panel with watertight seals. • Functions: Timer, revolution counter, and thermometer with heart probe. |
| Dimensions & Weight | • Width: 750 – 800 mm (Target: 770 mm) . • Depth: 950 – 1050 mm (Target: 1000 mm). • Height: 1700 – 1800 mm (Target: 1750 mm). • Weight: 280 – 320 kg (Target: 300 kg). |
| Mandatory Accessories (Included per machine) | To ensure full functionality and versatility as requested: • Standard Tools: 1x Spiral, 1x Blade/Paddle, 1x Whisk. • Extra Bowls: 2 additional stainless steel bowls (Total 3 bowls per machine). • Bowl Trolley: Trolley for safe transport of heavy bowls. • Scraper: Bowl scraper attachment for efficient mixing. • Reduction Kit: 40L or similar reduction kit (bowl + 3 tools) for smaller batches. • Air Injection System: For aerating mixtures. • Water Nebulisation System: For moisture control. • Lighting: Bowl lighting system. • Drain Cap: Bowl drain cap for cleaning. |
| Documentation | To be supplied in Latvian or English: user manual, electric diagram, maintenance instructions, CE Declaration of Conformity. |
| Compliance | Equipment must comply with EU Machinery Directive 2006/42/EC and food contact materials legislation. CE marking mandatory. |
| Guarantee | Minimum 24 months full manufacturer or supplier warranty. |

Name, surname, position

signature