# Annex No. 2

TECHNICAL SPECIFICATION

PURCHASE OF AUTOMATIC THERMOFORMING VACUUM PACKAGING LINE

place, date

| Parameter | Requirement |
| --- | --- |
| THE PARTICIPANT shall provide information or proof against each requirement. |  |
| Quantity | 1 pc. – Automatic thermoforming vacuum packaging line for RTE meals, smoked fish and soups, supporting vacuum, MAP and skin packaging. |
| Brand, model offered | Brand name (for identification purposes) and model name (for identification purposes). |
| Purpose | Thermoforming packaging line suitable for: C-PET trays (skin/vacuum) 250–300 g; A-PET trays (MAP) 200–500 g; C-PET vacuum/skin packs for smoked fish; round soup cups 150–200 g (deep-draw, sealed top film). |
| Output capacity | Minimum 10,000 retail packs per 8-hour shift depending on format. THE PARTICIPANT must provide per-format capacity table. |
| Overall dimensions (L × W × H) | Total line length 9,000–13,000 mm; width 1,100–1,400 mm; height 1,800–2,200 mm. Exact dimensions must be specified. |
| Hygienic design | Stainless steel AISI 304 minimum; wash-down compatible; sloped surfaces; machine housing min. IP65. |
| Chain / web width | 420–460 mm, suitable for 3–4 track layouts. |
| Forming film type and forming depth | Rigid thermoformable films (C-PET, A-PET). Adjustable forming depth 20–120 mm or greater. |
| Forming station | Must include sandwich pre-heating (top and bottom) and plug-assist forming for deep trays and soup cups. |
| Sealing station | Drawer-type quick-change sealing system enabling vacuum, MAP and skin sealing. |
| Film cooling module | Integrated cooling section before cutting for stabilising the top film after sealing. |
| Packaging technologies | Must support vacuum, MAP (gas flushing), and skin sealing on rigid trays. |
| Vacuum pump set | Centralised or machine-integrated system with rotary vane pump + booster (Roots type) for rapid evacuation. Supplier must specify configuration. |
| Gas mixing unit (MAP) | Gas mixer/doser for O₂/N₂/CO₂ mixtures; adjustable MAP composition with monitoring. |
| Cutting systems | Cross-cut and longitudinal cutting; shape or strip punch for round cups; integrated trim waste removal. |
| Tooling / formats | Two full format sets required: |
|  | Rectangular tray format (RTE meals, fish); |
|  | Round/deep-cup format (soups). |
|  | Layout drawings must be provided. |
| Label applicator | Top and/or bottom label applicator compatible with rigid trays/cups. |
| Inline printer | Inline thermal-transfer (or equivalent) printer for date, LOT, barcodes, QR codes directly on top web. |
| Infeed conveyor (hygienic) | Stainless-steel open-frame infeed conveyor 2,500–3,500 mm; wash-down compatible; suitable for manual loading. |
| Outfeed conveyor and accumulation | Stainless-steel discharge conveyor and accumulation system (buffer conveyor or rotary table). |
| Control system | PLC/IPC system with touchscreen HMI, recipe control, alarms, and vacuum/MAP monitoring. |
| Safety | Compliance with EU Machinery Directive; guarding, interlocks, emergency stops, safety circuits. |
| Cleaning and hygiene | Suitable for intensive wet cleaning; smooth surfaces; minimal dirt traps; full access to forming and sealing stations. |
| Utilities | Power supply 400 V, 3~ (3+N), 50 Hz. Compressed air, cooling and gas consumption must be specified. |
| Complete functional set (mandatory) | Offer must include all components required for complete functionality: thermoformer, forming station, sealing station, two tool sets, vacuum + booster system, MAP gas mixer, film cooling module, inline printer, label applicator, infeed conveyor, outfeed conveyor with accumulation, safety guarding, control system. Full list of included components required. |
| Supplier responsibility for task compliance | THE PARTICIPANT must offer such configuration and completeness that fully meets the purpose and technical task, including all necessary sub-components for full operation. |
| Documentation | User manual, installation and maintenance manuals, electrical/pneumatic diagrams, format change-over documentation, vacuum/gas system documentation, declaration of conformity (LV or EN). |
| Compliance and CE marking | Must comply with EU Machinery, EMC, LVD directives; CE marking mandatory. |
| Guarantee | Minimum 24-month complete warranty for the line and all included components. |