Annex No. 2

TECHNICAL SPECIFICATION

PURCHASE OF AUTOMATIC TILTING CYLINDRICAL BOILING PAN WITH MIXER

place, date

The following table:

| Parameter | Requirement |
| --- | --- |
| THE PARTICIPANT shall provide information or proof against each requirement. |  |
| Quantity | 2 pcs. – Industrial automatic tilting boiling pan with integrated mixer and PLC control. |
| Brand, model offered | Brand name (for identification purposes) and model name (for identification purposes). |
| Purpose | Large capacity industrial cooking (boiling, mixing) for mass catering. The unit must feature an automatic mixer and a tilting system designed to unload food into trolleys while maintaining a constant outlet height. |
| Capacity | • Tank Capacity: 280 – 320 litres. |
| Construction | • Material: Double-walled vessel made of stainless steel (AISI-304/316). • Design: Cylindrical structure with a high discharge point suitable for 200L trolleys. • Hygiene: Certified IPX5 protection or equivalent. |
| Heating System | • Type: Indirect Electric heating. • Technology: Heated by armoured heating elements immersed in the double jacket water/oil bath. |
| Mixer System | • Type: Integrated automatic mixer with adjustable speed. • Blades: Mixing blades must be customizable and easy to remove for cleaning. |
| Tilting Mechanism | • Type: Motorised automatic tilting. • Functionality: The pouring lip/outlet must remain at the same height during the tilting process to facilitate safe unloading into trolleys. |
| Control System | • Interface: Advanced PLC monitor with a Touch Screen. • Functions: Automation of cooking parameters, recipe management, mixer control, and tilting control. |
| Power & Voltage | • Total Electric Power: 35 kW – 50 kW. • Voltage: 400V / 3 ph / 50 Hz. |
| Dimensions | • Length: 1500 – 1900 mm. • Depth: 1550 – 1650 mm. • Height: 1100 – 1300 mm. |
| Mandatory Accessories (Included) | The supplier is responsible for providing a fully functional unit with the following specific accessories included: • Shower kit / Flexible hose for cleaning. • Automatic device for filling the tank with water. • Electronic water meter / Automatic water charge control system. • Filter/Strainer for the pouring lip. |
| Documentation | To be supplied in Latvian or English: user manual, electric diagram, maintenance instructions, CE Declaration of Conformity. |
| Compliance | Equipment must comply with EU Machinery Directive 2006/42/EC and food contact materials legislation. CE marking mandatory. |
| Guarantee | Minimum 24 months full manufacturer or supplier warranty. |

Name, surname, position

signature