# ANNEX No. 2

TECHNICAL SPECIFICATION

PURCHASE OF BAKERY SHOCK FREEZER (600×400 TRAYS, 20 LEVELS)

place, date

| Parameter | Requirement |
| --- | --- |
| THE PARTICIPANT shall provide information or proof against each requirement. |  |
| Quantity | 1 pc. – Industrial blast chiller for bakery/pastry production using 600×400 mm baking trays. |
| Brand, model offered | Brand name (for identification purposes) and model name (for identification purposes). |
| Purpose | Blast chiller must rapidly cool and freeze bakery/pastry products after baking. Must support chilling from +90°C to +3°C and freezing from +90°C to –18°C using intensive airflow and controlled freezing cycles. Positive chilling cycle capacity: ≥ 85 kg. Negative blast freezing cycle capacity: ≥ 60 kg |
| Tray type compatibility | Must be compatible exclusively with 600×400 mm baking trays (not GN 1/1 steam pans). Minimum 20 trays per loading cycle. |
| Overall external dimensions (L × W × H) | Dimensions must fall within the following ranges: |
|  | • length/depth 900–1900 mm, |
|  | • width 700–1500 mm, |
|  | • height 1800–2450 mm. |
| Internal chamber | Internal dimensions must allow insertion of racks/trolleys with 600×400 bakery trays. The unit must accept a mobile 20-level trolley up to 720×900×1880 mm. Rounded internal corners required for hygiene. Supplier must specify internal chamber dimensions. |
| Construction | Monoblock structure from stainless steel minimum AISI 304 with scotch-brite satin finish. |
| Insulation | Insulation thickness must be not less than 70 mm, high-density polyurethane foam (~40 kg/m³). |
| Airflow system | Frontal or directed air circulation ensuring uniform distribution for bakery trays across all 20 levels. |
| Freezing/chilling performance | Positive cycle (cooling) and negative cycle (freezing) must support manufacturer-certified performance for bakery products. Supplier must state certified kg per cycle. |
| Temperature range | +90°C to –18°C operating capacity. |
| Standard equipment | Must include: |
|  | • heated core probe |
|  | • removable magnetic gasket |
|  | • door micro-switch (fan stops when door opens) |
|  | • adjustable stainless-steel feet |
|  | • accessible evaporator compartment |
|  | • hot-gas or equivalent high-efficiency defrost system |
| Refrigeration system | Air-cooled or water-cooled remote condensing unit, OR integrated air condensing unit. Supplier must specify type and capacity. |
| Refrigerant | Acceptable refrigerants: R452A, R290, or equivalent EU-compliant low-GWP option. |
| Power supply | 400V 3~ or 400V 3+N 50Hz. Total power consumption must be indicated. |
| Hygiene and cleaning | Chamber must allow easy cleaning; evaporator must be accessible. Drainage must be integrated. |
| Safety features | Emergency stop, thermal protections, door safety switch, compliance with EU industrial safety standards. |
| Installation | Unit must be delivered fully assembled, requiring only electrical and refrigerant/drain connection. |
| Complete functional set (mandatory) | Offer must include: |
|  | • blast chiller chamber |
|  | • tray supports or rack system for 600×400 trays |
|  | • heated core probe |
|  | • airflow/evaporator system |
|  | • remote or integrated condensing unit (depending on configuration) |
|  | • any accessories necessary for full operational function |
| THE PARTICIPANT must provide a complete list of all included components. |  |
| Documentation | To be supplied in Latvian or English: user manual, installation manual, technical passport, maintenance instructions, wiring diagram, declaration of conformity. |
| Compliance and CE marking | Must comply with EU Machinery Directive, EMC, LVD and refrigerant-related legislation. CE marking mandatory. |
| Guarantee | Minimum 24 months full manufacturer or supplier warranty. |

Name, surname, position

signature