Annex No. 2

TECHNICAL SPECIFICATION

PURCHASE OF COMPACT SMOKING CHAMBER (FISH & MEAT PRODUCTS)

place, date

The following table:

| Parameter | Requirement |
| --- | --- |
| THE PARTICIPANT shall provide information or proof against each requirement. |  |
| Quantity | 1 pcs. – Compact automatic smoking chamber for craft production, fully equipped for fish processing. |
| Brand, model offered | Brand name (for identification purposes) and model name (for identification purposes). |
| Purpose | Universal thermal processing facility designed for craft production in smaller plants. The unit must be suitable for automatic processing of smoked meat products, poultry, fish, and cheese. |
| Technological Processes | The chamber must automatically perform the following phases: reddening, warming-through, drying, smoking (hot and cold), cooking, baking, and showering. |
| Construction & Hygiene | • Material: All-stainless steel construction (CrNi), including the frame and paneling. • Insulation: Chamber walls insulated with mineral rock wool or equivalent material to prevent heat loss. • Design: Compact design, preferably with the smoke generator integrated into the door to save floor space. |
| Capacity | • Trolley Capacity: Designed to accommodate 1 smoking trolley (dimensions minumum 1000x1000x2000 mm). • Product Load: 60 kg – 150 kg per cycle, depending on the product type (fish/meat). |
| Smoke Generator | • Type: Automatic wood chip smoke generator. • Integration: Integrated into the chamber door or compact body to minimize installation footprint. • Control: Microprocessor-controlled ignition and smoke density. |
| Air Circulation System | • Fan: Powerful radial or circular fan ensuring uniform temperature and smoke distribution. • Speed Control: Speed variation (frequency converter) to adapt airflow for sensitive products like fish. |
| Control System | • Interface: Microprocessor control unit with a Touch Screen or foil keyboard. • Functionality: Storage for 80 – 100 recipes/programs; automatic control of temperature, humidity, and time; data registration. |
| Temperature & Humidity | • Chamber Temperature: Adjustable range up to 100°C – 110°C. • Core Temperature: Measured via piercing core sensor. • Humidity: Controlled via psychrometric sensor. |
| Cleaning System | • Type: Automatic washing system capable of cleaning the interior, air distribution system, and smoke generator lines. |
| Heating System | • Type: Electric heating (stainless steel heating bodies). • Power Input: 24 kW – 28 kW. • Voltage: 400V / 3 ph / 50 Hz. |
| Dimensions | • Width: 1200 mm – 1350 mm. • Depth: 1000 mm – 1150 mm. • Height: 2200 mm – 2400 mm. |
| Complete Functional Set (Mandatory) | The supplier is responsible for providing a fully functional unit with all necessary components for smooth operation of the specific task (Fish Smoking), including but not limited to: • 1 x Stainless Steel Smoking Trolley: Specifically equipped for fish (e.g., with appropriate screens/racks or hooks depending on fish type stated in project). • Smoke Sticks/Grids: One full set for the trolley. • Smoke Generator Wood Chips: Starter pack. • Cleaning Agent: Starter pack for the automatic cleaning system. • Core Temperature Probe. • Installation: Delivery, assembly, and commissioning included. |
| Documentation | To be supplied in Latvian or English: user manual, electric diagram, maintenance instructions, CE Declaration of Conformity. |
| Compliance | Equipment must comply with EU Machinery Directive 2006/42/EC and food contact materials legislation. CE marking mandatory. |
| Guarantee | Minimum 24 months full manufacturer or supplier warranty. |

Name, surname, position

signature