# Annex No. 2

TECHNICAL SPECIFICATION

CONICAL DOUGH ROUNDING MACHINE (DOUBLE-CONE TYPE)

Place, date: ____________________

| Parameter | Requirement |
| --- | --- |
| Quantity | 1 pc – Industrial conical dough rounding machine (double-cone type) for rounding dough pieces for bakery production. |
| Purpose | Suitable for rounding dough pieces for bread rolls and similar products; must ensure uniform shape without deformation, including soft doughs. |
| Dough piece weight range | Must handle dough pieces within 100–2,000 g. |
| Construction | Food-grade coated aluminium channels and food-grade cone (non-toxic nickel or Teflon). Stainless-steel supporting frame. |
| Machine type | Double-cone rounding system with automatic transfer from first to second cone. |
| Installed power | Total installed electrical power must fall within 0.7–1.6 kW. |
| Electrical supply | 400 V, 3~ 50 Hz. |
| Flour dusting system | Integrated adjustable flour duster. |
| Cone & channel material | Cone must be finished with non-toxic nickel or Teflon coating; channels must be made of coated aluminium with non-stick properties. |
| Mobility | Machine must be mounted on wheels for operational mobility. |
| Rounding performance | Machine must provide continuous rounding suitable for industrial bakeries; THE PARTICIPANT must declare dough throughput per hour. |
| Dimensions (L × W × H) | Machine dimensions must fall within the following ranges: |
|  | Length 1200–1400 mm |
|  | Width 900–1050 mm |
|  | Height 1600–1800 mm |
| Weight | Minimum 550 kg. |
| Optional functions (may be included) | THE PARTICIPANT shall declare inclusion of: |
|  | – Exit conveyor belt (min. length 600 mm) |
|  | – Warm/cold air blowing channels with adjustable thermostat |
|  | – Stainless-steel external covers |
| Safety & hygiene | Machine must comply with EU food-contact and machinery safety requirements; smooth surfaces, easy cleaning, hygiene-compliant design. |
| Documentation | User manual, maintenance instructions, spare parts list, CE documentation. Provided in EN or LV. |
| Compliance / CE marking | Must comply with EU Machinery Directive, EMC Directive, food-contact requirements; CE marking mandatory. |
| Guarantee | Minimum 24-month warranty for the complete machine. |
| Supplier responsibility for functionality | THE PARTICIPANT must offer such configuration and completeness that fully meets the purpose and technical task, including all necessary sub-components for full operation. |

Name, surname, position

signature