# Annex No. 2

TECHNICAL SPECIFICATION

PURCHASE OF CONTINUOUS PASTA COOKER (WITH AUTOMATIC LIFTING BASKET)

place, date

## Parameter Table (Compliance Required)

| Parameter | Requirement |
| --- | --- |
| Quantity | 1 pc. – Continuous pasta cooker with automatic basket lifting, double-tank boiling system, stainless-steel construction, for pasta/noodles cooking in industrial CPU workflow. |
| Brand, model offered | Brand name (identification only) and model name (identification only). |
| Purpose | Continuous boiling of pasta, noodles, dumplings and similar products, with automatic lifting system, timed immersion, starch overflow drainage, and automatic water reintegration for industrial RTE food production. |
| Boiling tank configuration | Two independent tanks (2 pcs.). EACH tank net working capacity must be between 200–240 litres per tank. |
| Total usable volume | Combined usable capacity (both tanks) must be between 400–480 litres total. |
| Overall dimensions (L × W × H) | Total dimensions must fall within these limits: |
|  | • Length: 2400–2600 mm |
|  | • Width: 900–1100 mm |
|  | • Height: 900–1100 mm |
|  | Exact dimensions of offered model must be specified. |
| Construction material | Stainless steel AISI 304 or higher for all food-contact and external surfaces. Baskets made from AISI 316 or higher. |
| Heating system (electric) | Total electrical heating power must fall between 40–60 kW (combined for both tanks). |
| Temperature range | Adjustable temperature range 50–120°C. |
| Controls | Digital control panel with minimum: |
|  | • independent tank temperature adjustment |
|  | • digital time programming (submersion time adjustable ≥1 sec steps) |
|  | • cycle completion acoustic/light signal |
| Automatic basket lifting | Mandatory: automatic raising/lowering system with programmable immersion times. |
| Basket draining logic | System must allow: |
|  | • first drip position |
|  | • full unload |
|  | • option for direct front unload |
| Drainage system | Integrated overflow and starch removal outlet; full drainage tap per tank. |
| Fresh-water reintegration | Automatic delayed water reintegration at end of cooking cycle. |
| Safety features | Overheat protection thermostat, low-water protection, safety valve, thermocouple, emergency stop. |
| Voltage | 400 V, 3~ (3+N), 50 Hz. |
| Inclusions (mandatory) | The cooker must include: |
|  | • 2 × lifting baskets |
|  | • digital time–temperature controllers |
|  | • full drainage set |
|  | • fresh-water reintegration |
|  | • overflow starch drainage system |
|  | • adjustable stainless-steel feet |
|  | • steam/smoke extraction interface or hood-ready design |
| Cleaning and hygiene | Smooth, rounded corners; full access for cleaning; tank lids with safe lifting; IPX water-resistant design suitable for wet industrial cleaning. |
| Operating environment | Must support continuous multi-cycle production typical to a central kitchen (CPU) with high humidity and frequent cleaning. |
| Noise level | ≤70 dB at 1 m distance. |
| Utilities | Supplier must specify: |
|  | • electrical loading |
|  | • water supply and drainage requirements |
|  | • any optional steam/extraction requirements |
| Documentation | User manual, installation manual, maintenance manual, electrical diagrams, declaration of conformity (LV or EN). |
| Compliance & CE marking | Must comply with EU Machinery Directive, EMC, LV, Hygiene Regulations (HACCP-compatible). CE marking mandatory. |
| Warranty | Minimum 24-month full warranty for complete equipment and all components. |
| Supplier responsibility for task compliance | THE PARTICIPANT must offer such configuration and completeness that fully meets the purpose and technical task, including all necessary sub-components for full operation. |
| Complete functional set (mandatory) | Offer must include all components required for complete functionality: full cooker body, dual tanks, baskets, lifting motors, lids, drainage, control system, fresh-water reintegration, safety systems and all required accessories for industrial operation. |

Name, surname, position

signature