# Annex No. 2

# TECHNICAL SPECIFICATION

# PURCHASE OF  DOUGH SHEETER (FLOOR TYPE) WITH FULL CUTTING ACCESSORY SET

place, date

| Parameter | Requirement |
| --- | --- |
| Quantity | 1 pc. – Industrial dough sheeter for pastry, croissants, fresh pasta and laminated dough, with full cutting and shaping accessory package. |
| Brand, model offered | Brand name (for identification purposes) and model name (for identification purposes). |
| Purpose | Floor-type dough sheeter for producing thin, homogeneous dough sheets, including croissant, puff pastry, phyllo, pasta, shortcrust and similar products. Must include complete roller cutting set and pasta kit. |
| Installed power (kW) | From 0.70 to 1.00 kW. THE PARTICIPANT must specify exact installed power. |
| Weight (kg) | From 250 to 350 kg depending on configuration. THE PARTICIPANT must specify exact weight. |
| Voltage | 400 V / 3~ / 50 Hz (or 220 V / 1~ / 50 Hz if CE-compliant). THE PARTICIPANT must specify exact voltage. |
| Conveyor belt dimensions (mm) | Working belt width 585–600 mm. Belt length per side 1150–1600 mm. THE PARTICIPANT must specify exact belt dimensions. |
| Overall dimensions (mm) | Arms opened: 3000×1000×1200 mm to 3400×1100×1300 mm. Arms folded: 1300×1000×1300 mm to 1600×1100×1300 mm. Exact L×W×H must be specified. Overall dimensions (mm), arms opened (operating position): Length: 3000–3500 mm Width: 1000–1100 mm Height: 1200–1300 mm Exact L × W × H of the offered model must be specified by the Participant. |
| Rolling cylinders | Hardened and chrome-polished cylinders, Ø70–80 mm. Adjustable opening range 0.3–48 mm. |
| Hygienic design | Stainless-steel or anticorodal aluminium shoulders; removable upper and lower scrapers; smooth surfaces; ball bearings; CE-compliant safety protections; machine on wheels with locking brakes. |
| Reversibility & speed control | Reversible sheeting direction; belts with multi-speed control; inverter or equivalent speed-regulation technology. |
| Mandatory included cutting accessories | THE PARTICIPANT must include ALL: Croissant triangle-cut roller set, strip-cutting rollers, round- and square-cut rollers, fresh-pasta cloth-belt kit, universal reversible scrapers, exit conveyor belt (min. 600 mm), warm/cold air blowing channels with thermostat. Cutting and forming accessories: The offered dough sheeter must be compatible with and supplied with a complete cutting accessory set suitable for croissants, laminated dough and fresh pasta production, including: – croissant triangle-cut roller, – strip-cutting rollers, – straight cutting rollers, – fresh-pasta cloth-belt kit, – reversible scrapers. All accessories must be supplied within the scope of delivery. |
| Additional included options (mandatory) | Stainless steel external coating; complete roller cutting kit; rising and dismountable arms; low-tension control panel; ball-bearing movement system. |
| Safety | Compliance with EU Machinery Directive; CE marking; integrated emergency-stop devices; operator-safety guarding. |
| Cleaning and hygiene | Removable scrapers without tools; easy-to-clean conveyors; flour duster with adjustable flow. |
| Utilities | THE PARTICIPANT must specify electrical consumption, air requirements (if any), and all additional utilities. |
| Complete functional set (mandatory) | THE PARTICIPANT must include all components required for full operation: base machine, reversible cylinders, complete cutting kit, croissant cutting group, fresh-pasta kit, exit conveyor, air channels, scrapers, safety guarding, documentation. |
| Supplier responsibility for task compliance | THE PARTICIPANT is responsible for offering such configuration and completeness that fully meets the stated purpose and technical task, including all sub-components necessary for complete functionality. |
| Documentation | User manual; installation manual; maintenance instructions; electrical diagrams; CE declaration of conformity. |
| Guarantee | Minimum 24-month full warranty for the complete machine and all included components. |

Name, surname, position

signature