# Annex No. 2

TECHNICAL SPECIFICATION

ELECTRIC CREAM COOKING KETTLE (BAIN-MARIE TYPE)

### Place, date: ___________________________

| Parameter | Requirement |
| --- | --- |
| Quantity | 1 pc. – Electric cream cooking kettle, bain-marie type, suitable for cooking creams, sauces, jams, fillings, condensed milk, béchamel, risotto, polenta and similar products. |
| Purpose / application | Industrial kettle for heat-sensitive products requiring uniform temperature, scrapers and bain-marie oil system. Must support continuous operation. |
| Working principle | Triple-wall cooking bowl with diathermic oil (“bain-marie”) heating system ensuring stable, homogeneous heat without product burning. |
| Bowl capacity | Nominal bowl volume within 55–65 L; usable product capacity 40–50 L. |
| Construction material | All food-contact and main structural components in AISI 304 stainless steel or higher. |
| Mixing system | Central mixing tool with bottom and side scrapers. Mixing direction must be reversible (clockwise and counterclockwise). |
| Mixing speed | Two-speed motor or variable-speed electronic control. THE PARTICIPANT shall declare the actual range. |
| Heating method | Electric heating elements immersed in diathermic oil. Installed heating power to fall within 5.5–7.5 kW. |
| Temperature control | Electronic thermostat; adjustable temperature range 20–120 °C. |
| Safety systems | Overheat protection (oil), low-oil protection, mixing tool guard, CE-compliant interlocks. |
| Discharge / emptying | Machine must include integrated discharge solution: |
|  | a) tilting bowl, or |
|  | b) bottom discharge valve (food-grade ball valve). |
|  | THE PARTICIPANT must clearly specify the included solution. |
| Bowl mobility | Horizontally pivoted removable bowl or fixed bowl with valve discharge. |
| Electrical supply | 400 V, 3~, 50 Hz. THE PARTICIPANT must declare total power load. |
| Dimensions (L × W × H) | Must fall within: |
|  | Length 760–1450 mm |
|  | Width 700–840 mm |
|  | Height 700–1500 mm |
| Weight | Minimum 120 kg. THE PARTICIPANT must specify actual weight. |
| Control panel | Integrated electronic controls: heating thermostat, timer, mixing control, safety indicators. |
| Cleaning & hygiene | Smooth internal surfaces; removable scrapers; hygienic stainless-steel structure; full access for cleaning. |
| Noise level | Must comply with EU limits for food equipment. THE PARTICIPANT must declare values. |
| Included components (mandatory) | Thermic oil filled and installed; safety lid; scraper set; reversible mixing; control electronics; stainless-steel support frame. THE PARTICIPANT must specify full list. |
| Optional functions (required unless stated non-applicable) | Variable speed, tilting mechanism, valve discharge, wheels, dosing port. Supplier must specify which are included. |
| Documentation | User manual, maintenance manual, wiring diagrams, CE Declaration of Conformity. Provided in EN or LV. |
| Compliance / CE | Must comply with EU Machinery Directive, EMC Directive and LVD; CE marking mandatory. |
| Guarantee | Minimum 24-month full warranty. |
| Supplier responsibility for full functionality | THE PARTICIPANT must offer such configuration and completeness that fully meets the purpose and technical task, including all necessary sub-components for full operation. |

Name, surname, position

signature