Annex No. 2

TECHNICAL SPECIFICATION

PURCHASE OF ELECTRIC TILTING CYLINDRICAL BOILING PAN

place, date

The following table:

| Parameter | Requirement |
| --- | --- |
| THE PARTICIPANT shall provide information or proof against each requirement. |  |
| Quantity | 2 pcs. – Industrial electric tilting cylindrical boiling pan. |
| Brand, model offered | Brand name (for identification purposes) and model name (for identification purposes). |
| Purpose | Large capacity industrial cooking (boiling) for mass catering. The unit must feature a cylindrical design and motorised tilting for easy unloading. |
| Capacity | • Tank Capacity: 190 – 210 litres. |
| Construction | • Material: Double-walled vessel. Well bottom made of AISI-316 stainless steel (thickness 20/10 – 40/10) for high corrosion resistance. Walls made of AISI-304 . • Design: Cylindrical structure suitable for placement in transit areas, allowing use from different angles. |
| Heating System | • Type: Indirect Electric heating. • Technology: Heated by shielded/armoured heating elements immersed in the double jacket water bath to generate steam. Includes pressure control via pressure switch, safety valve (0.5 bar), depression valve, and gauge55. |
| Tilting Mechanism | • Type: Motorised automatic tilting of the pan for safe emptying. |
| Control System | • Type: Electronic control. • Temperature: Adjustable range approx. 50°C – 120°C. |
| Power & Voltage | • Total Electric Power: 34 kW – 38 kW. • Voltage: 400V / 3 ph / 50 Hz. |
| Dimensions | • Length: 1300 – 1350 mm. • Depth: 1250 – 1300 mm. • Height: 1000 – 1100 mm. |
| Mandatory Accessories (Included) | The supplier is responsible for providing a fully functional unit with the following specific accessories included: • Shower kit / Flexible hose for cleaning. • Automatic device for filling the tank with water. • Electronic water meter / Automatic water charge control system. • Filter/Strainer for the pouring lip . |
| Documentation | To be supplied in Latvian or English: user manual, electric diagram, maintenance instructions, CE Declaration of Conformity. |
| Compliance | Equipment must comply with EU Machinery Directive 2006/42/EC and food contact materials legislation. CE marking mandatory. |
| Guarantee | Minimum 24 months full manufacturer or supplier warranty. |

Name, surname, position

signature