Annex No. 2

TECHNICAL SPECIFICATION

PURCHASE OF ELECTRICALLY HEATED THERMO TRANSPORT BOXES (DOUBLE COMPARTMENT)

place, date

The following table:

| Parameter | Requirement |
| --- | --- |
| THE PARTICIPANT shall provide information or proof against each requirement. |  |
| Quantity | 40 pcs. – Double compartment electrically heated food transport boxes with full accessory kit. |
| Brand, model offered | Brand name (for identification purposes) and model name (for identification purposes). |
| Purpose | Transportation and holding of hot food in Gastronorm (GN) containers. The unit must feature two independent electrically heated compartments with digital temperature control. Designed for professional catering and "cook-serve" logistics. |
| Capacity | • Configuration: Two vertically stacked compartments. • Capacity per Compartment: 12 – 16 x GN 1/1 (depending on depth, standard spacing ~80 mm). • Total Capacity: 24 – 32 x GN 1/1. |
| Temperature Control | • Heating System: Active electric heating (convection or foil/static) in both compartments. • Control: Digital thermostat with temperature display for each compartment. • Range: Adjustable up to +70°C – +90°C2. |
| Construction & Insulation | • Interior: Fully welded stainless steel container with pressed guides (no joints/rivets) for maximum hygiene . • Insulation: Polyurethane foam (PU) expanded under high pressure. • Exterior: Durable ABS plastic or similar material. • Protection Class: Minimum IP44 or IP55. |
| Dimensions | • Width: 900 mm – 950 mm • Depth: 750 mm – 800 mm • Height: 1300 mm – 1400 mm |
| Weight | • Net Weight: 100 kg – 120 kg. |
| Electrical Connection | • Voltage: 230V / 1-phase / 50 Hz. • Power by each department: up to 2000 W (depending on heating type E or F). |
| Accumulators (Batteries) | Must include Thermal Accumulators (Eutectic Plates) for maintaining temperature during transport when disconnected from mains, or equivalent battery backup solution if available. |
| Mandatory Accessories (Included) | The supplier is responsible for providing a fully functional set with the following specific options included: • Wheels: up to Ø 200 mm castors, 2 fixed and 2 swivel with brakes. • Towing: Towing set for linking boxes. • Brake: Central brake system compatible with the wheels. • Labeling: Label holder A5 size. • Storage: Tray holder / Rack on top/side. • Handling: Lockable one-grip handle. • Power: Extra long connection cable, approx. 5-6 meters. |
| Documentation | To be supplied in Latvian or English: user manual, maintenance instructions, CE Declaration of Conformity. |
| Compliance | Equipment must comply with EU Machinery Directive, Low Voltage Directive, and food contact materials legislation (EN 631). CE marking mandatory. |
| Guarantee | Minimum 24 months full manufacturer or supplier warranty. |

Name, surname, position

signature