# Annex No. 2

TECHNICAL SPECIFICATION

PURCHASE OF FULLY AUTOMATIC VEGETABLE WASHING AND PEELING SYSTEM

place, date

| Parameter | Requirement |
| --- | --- |
| THE PARTICIPANT shall provide information or proof against each requirement. |  |
| Quantity | 1 pc. – Fully automatic vegetable washing and peeling system suitable for potatoes, carrots, beetroots and similar root vegetables. |
| Brand, model offered | Brand name (for identification purposes only) and model name (for identification purposes only). |
| Purpose | Industrial system for washing, peeling, discharge and wastewater removal, suitable for potatoes, carrots and beetroots. |
| Batch loading capacity | From 25 kg to 40 kg per cycle. |
| Processing capacity | From 800 kg/h to 1,100 kg/h. THE PARTICIPANT must provide processing table. |
| Overall dimensions (W × D × H) | Width 800–950 mm; depth 900–1200 mm; height 1150–1350 mm. |
| Construction | Stainless steel AISI 304 minimum; AISI 316 for critical parts; hygienic rounded corners. |
| Ingress protection | Minimum IP66. |
| Automation | Automatic control of filling, washing, peeling, discharge and drainage; 1‑second time adjustment; restart protection. |
| Peeling technologies | Blade peeling with reversible rotation and double‑edged blades; carborundum friction peeling. Both must be provided. |
| Gentle handling | Water‑cushion or equivalent system; gentle low‑speed release. |
| Discharge system | Automatic or semi‑automatic discharge with reversible rotation. |
| Safety features | Lid/flap safety switches; overload and motor protection; emergency stop; restart block. |
| Electrical power | 1.2–2.2 kW. |
| Electrical supply | 400 V, 3~ (3+N), 50 Hz. |
| Water supply | DN 20–DN 32 with controlled inlet. |
| Drainage | Integrated wastewater drainage system. |
| Included accessories | Smooth washing insert; brush insert; blade peeling insert; carborundum insert; peel collector; starch separator; drain kit; installation accessories. |
| Control system | Digital panel with status display, time programming, start/stop, end‑cycle signalling. |
| Cleaning & maintenance | Removable internal parts; external wet cleaning allowed. |
| Utilities | Supplier specifies electrical and water consumption and drainage needs. |
| Complete functional set | All components required for complete functionality must be included. |
| Supplier responsibility | Supplier must offer configuration fully meeting purpose and technical task. |
| Documentation | User manual; installation manual; maintenance manual; electrical diagrams; declaration of conformity. |
| Compliance and CE marking | EU Machinery, EMC, Low Voltage Directives; CE marking mandatory. |
| Guarantee | Minimum 24‑month warranty. |

Name, surname, position

signature