Annex No. 2

TECHNICAL SPECIFICATION

PURCHASE OF GENTLE TILTABLE MIXER

place, date

The following table:

| Parameter | Requirement |
| --- | --- |
| THE PARTICIPANT shall provide information or proof against each requirement. |  |
| Quantity | 2 pcs. – Industrial tiltable mixer for salads, meat products, and deli mixtures. |
| Brand, model offered | Brand name (for identification purposes) and model name (for identification purposes). |
| Purpose | Gentle and intensive mixing of various products (salads, meat, vegetables, delicatessen) with a focus on maintaining product integrity. Suitable for standardizing raw materials and mixing ingredients evenly. |
| Mixing System | • Type: Dual-shaft mixing system with intermeshing paddle shafts or Z-arms. • Action: Large effective operating range due to intermeshing shafts, ensuring fast, uniform, and gentle mixing without crushing delicate ingredients (like salad leaves). • Speed: Infinitely variable mixing speed to adapt to different product consistencies. |
| Capacity | • Hopper Volume: 120 – 140 litres. • Max. Filling/Mixing Capacity: 100 – 120 kg. |
| Discharge / Draining | • Method: Tiltable hopper (hydraulic or electric tilting mechanism) for quick and complete emptying. • Safety: Automatic safety shutdown during tilting or opening of the safety cover. |
| Construction | • Material: Entire machine frame, hopper, and mixing shafts made of solid stainless steel (AISI 304). • Design: Compact, self-supporting housing with integrated control cabinet to save space. Double sealing of mixing shaft ends for hygiene. |
| Motor & Power | • Power: 3.5 kW – 5.5 kW. • Voltage: 400V / 3 ph / 50 Hz. |
| Dimensions | • Length: 1100 – 1400 mm. • Width: 700 – 950 mm. • Height: 1100 – 1350 mm (excluding lid opening). |
| Control System | • Type: Push-button control or Microprocessor control with digital display. • Functions: Mixing time pre-selection, speed adjustment, forward/reverse operation. |
| Complete Functional Set (Mandatory) | The supplier is responsible for providing a fully functional unit with all necessary components for immediate operation, including: • Stainless steel mixing hopper (tiltable) • 2 x Universal mixing shafts (paddles) • Safety grid/cover with auto-stop • Integrated electrical control panel • Installation and maintenance manual |
| Documentation | To be supplied in Latvian or English: user manual, electric diagram, maintenance instructions, CE Declaration of Conformity. |
| Compliance | Equipment must comply with EU Machinery Directive 2006/42/EC and food contact materials legislation. CE marking mandatory. |
| Guarantee | Minimum 24 months full manufacturer or supplier warranty. |

Name, surname, position

signature