Annex No. 2

TECHNICAL SPECIFICATION

PURCHASE OF GN 2/1 REFRIGERATED CABINET (DOUBLE DOOR)

place, date

The following table:

| Parameter | Requirement |
| --- | --- |
| THE PARTICIPANT shall provide information or proof against each requirement. |  |
| Quantity | 2 pcs. – Premium double-door vertical refrigerated cabinet (Advance range or equivalent). |
| Brand, model offered | Brand name (for identification purposes) and model name (for identification purposes). |
| Purpose | High-efficiency storage of fresh food products in Gastronorm 2/1 format. Suitable for intensive use in hot commercial kitchens (Climate Class 5, up to 40°C ambient). |
| Capacity | • Gross Volume: 1300 – 1400 litres • Net Volume: >950 litres. • Shelving: Designed for GN 2/1 shelves/grids. • Storage Levels: Embedded guides with approx. 23-24 levels (55 mm spacing) for maximum flexibility. |
| Performance & Temperature | • Working Temperature: 0°C to +8°C adjustable range. • Cooling System: Top-mounted "Monoblock" refrigeration unit with ventilated cooling (forced air). • Refrigerant: Environmentally friendly hydrocarbon R290 or R600a (GWP=3). • Energy Efficiency Class: C or better (A, B, C) according to EU energy labeling. • Climate Class: 5 (Heavy Duty, 40°C/40% RH). |
| Construction | • Material: Interior and exterior body made of high-quality Stainless Steel AISI-304. • Insulation: High-density polyurethane foam (40 kg/m³), thickness 80 mm (CFC-free) for superior thermal efficiency. • Doors: 2 x Self-closing solid stainless steel doors with full-length integrated handles. Stay-open feature at >90°. |
| Hygiene & Cleaning | • Interior: Deep-drawn/embedded tray guides (no separate rails/brackets to clean) and rounded bottom corners. • Drain: Integrated floor drain for discharging wash water. • Gaskets: Removable triple-chamber magnetic door gaskets. |
| Control System | • Interface: Advanced Digital control with 5-inch capacitive touch screen (or larger). • Functions: IPX5 protection, HACCP log/history, ECO mode for energy saving, visual and acoustic alarms. |
| Dimensions | • Width: 1340 mm – 1370 mm. • Depth: 840 mm – 870 mm. • Height: 2050 mm – 2150 mm. |
| Electrical Connection | • Voltage: 230V / 1 ph / 50 Hz. • Power: Low consumption approx. 180 – 220 W. |
| Complete Functional Set (Mandatory) | The supplier is responsible for providing a fully functional unit ready for immediate use. The set must include: • 1 x Double Door Refrigerated Cabinet • 6 x GN 2/1 Epoxy coated or Stainless steel grids/shelves (3 per door). • LED internal lighting. • Door locks. • Adjustable stainless steel legs (height adjustable). • Power cable and plug. |
| Documentation | To be supplied in Latvian or English: user manual, electric diagram, maintenance instructions, CE Declaration of Conformity. |
| Compliance | Equipment must comply with EU Machinery Directive 2006/42/EC, Low Voltage Directive, and food contact materials legislation. CE marking mandatory. |
| Guarantee | Minimum 24 months full manufacturer or supplier warranty. |

Name, surname, position

signature