.

Annex No. 2

TECHNICAL SPECIFICATION

PURCHASE OF GN 2/1 REFRIGERATED CABINET (SINGLE DOOR)

place, date

The following table:

| Parameter | Requirement |
| --- | --- |
| THE PARTICIPANT shall provide information or proof against each requirement. |  |
| Quantity | 4 pcs. – Vertical refrigerated cabinet (Concept range or equivalent) for professional catering. |
| Brand, model offered | Brand name (for identification purposes) and model name (for identification purposes). |
| Purpose | Preservation of fresh food products in Gastronorm 2/1 format containers or on shelves. Suitable for commercial kitchens with ambient temperatures up to +30°C (Climate Class 4). |
| Capacity | • Volume: Gross volume 550 – 650 litres. • Shelving: Designed for GN 2/1 shelves/grids. • Storage Levels: Embedded guides with min 23 levels (55 mm spacing) for maximum flexibility. |
| Performance & Temperature | • Working Temperature: 0°C to +10°C adjustable range. • Cooling System: Top-mounted refrigeration unit with ventilated cooling (forced air) for even temperature distribution. • Refrigerant: Environmentally friendly hydrocarbon R600a (GWP=3). • Energy Efficiency Class: C or better (according to EU energy labeling). |
| Construction | • Material: Interior and exterior body made of high-quality Stainless Steel (AISI-304/430 or equivalent). • Insulation: High-density polyurethane foam, thickness 60 mm (CFC-free). • Door: Self-closing solid stainless steel door with full-length ergonomic handle. Stay-open feature at >90°. |
| Hygiene & Cleaning | • Interior: Deep-drawn/embedded tray guides (no separate rails to clean) and rounded bottom corners for easy cleaning. • Drain: Integrated floor drain for discharging wash water. • Protection: IPX5 water protection rating. |
| Control System | • Interface: Digital electronic control with capacitive touch screen (min. 3 inches) and temperature display. • Functions: ECO mode for energy saving. |
| Dimensions | • Width: 680 mm – 710 mm ( • Depth: 710 mm – 750 mm • Height: 2000 mm – 2100 mm |
| Electrical Connection | • Voltage: 230V / 1 ph / 50 Hz. |
| Complete Functional Set (Mandatory) | The supplier is responsible for providing a fully functional unit ready for immediate use. The set must include: • 1 x Refrigerated Cabinet • 3 x GN 2/1 Epoxy coated or Stainless steel grids/shelves per cabinet. • Adjustable stainless steel legs (height adjustable 130-200 mm). • Power cable and plug. |
| Documentation | To be supplied in Latvian or English: user manual, electric diagram, maintenance instructions, CE Declaration of Conformity. |
| Compliance | Equipment must comply with EU Machinery Directive 2006/42/EC, Low Voltage Directive, and food contact materials legislation. CE marking mandatory. |
| Guarantee | Minimum 24 months full manufacturer or supplier warranty. |

Name, surname, position

signature