# Annex No. 2

TECHNICAL SPECIFICATION

PURCHASE OF HAND-WASH BASIN (KNEE-OPERATED, WALL-MOUNTED)

place, date

| Parameter | Requirement |
| --- | --- |
| THE PARTICIPANT shall provide information or proof against each requirement. |  |
| Quantity | 35 pcs. – Wall-mounted hand-wash basins, knee-operated type. |
| Brand, model offered | Brand name (for identification purposes) and model name (for identification purposes). |
| Purpose | Designed for personnel hygiene in food-processing areas; minimises contamination risk through hands-free operation. |
| Overall dimensions (W × D × H) | External dimensions must fall within: |
|  | • width: 400–500 mm |
|  | • depth: 300–400 mm |
|  | • height (incl. splashback): 450–650 mm |
|  | Supplier must specify the exact dimensions of model offered. |
| Mounting type | Wall-mounted construction (no floor support permitted). |
| Operation type | Knee-operated activation system; no hand-operated levers permitted. |
| Construction material | Stainless steel AISI 304 minimum. Optional upgrade to AISI 316L / AISI 316Ti must be available. |
| Basin depth (internal) | Minimum 150 mm internal depth. |
| Splashback | Integrated stainless-steel splashback with height minimum 200 mm. |
| Tap | Stainless steel water tap with hygienic design and jet regulator. |
| Water mixing valve | Integrated water mixing valve delivering 35 °C – 65 °C water. |
| Hygiene design | Smooth surfaces, rounded corners, no crevices; compliant with HACCP / IFS / NSF hygiene principles. |
| Delivered ready-to-install | Unit must be supplied fully assembled with wall brackets and ready for installation. |
| Accessories (optional) | Supplier may offer optional soap dispenser, paper towel holder, or utensil/knife steriliser. Optional items must be listed separately. |
| Installation requirements | Compatible with standard hot/cold water supply and drain connections for food-processing areas. |
| Complete functional set (mandatory) | Basins must include: |
|  | • knee-operated system |
|  | • mixing valve |
|  | • integrated splashback |
|  | • stainless-steel tap |
|  | • drain outlet |
|  | • wall-mounting kit |
|  | Supplier must provide complete list of included elements. |
| Documentation | User manual, installation instructions, maintenance instructions, declaration of conformity (LV or EN). |
| Compliance and CE marking | CE marking mandatory; compliance with EU hygiene and equipment safety regulations. |
| Guarantee | Minimum 24-month full warranty. |

Name, surname, position

signature