Annex No. 2

TECHNICAL SPECIFICATION

PURCHASE OF HEAVY-DUTY COMMERCIAL POWER MIXER

place, date

The following table:

| Parameter | Requirement |
| --- | --- |
| THE PARTICIPANT shall provide information or proof against each requirement. |  |
| Quantity | 1 pc. – Heavy-duty industrial immersion blender (Power Mixer) with pan support accessories. |
| Brand, model offered | Brand name (for identification purposes) and model name (for identification purposes). |
| Purpose | Intensive use in commercial catering and central kitchens for mixing soups, sauces, and purees in large quantities. The unit must be capable of processing quantities up to 400 litres. |
| Performance | • Processing Capacity: Up to 350 – 450 litres per pot. • Speed: Fixed speed operation in the range of 9,000 – 10,000 rpm to ensure fine texture in minimal time. |
| Motor & Power | • Power: High-power motor, range 950 W – 1100 W for intensive use. • Voltage: Single phase 230V / 50 Hz. |
| Construction & Hygiene | • Motor Base: Stainless steel construction for long service life and durability. • Sanitation: The bell and blade assembly must be fully removable for easy cleaning and maintenance to meet HACCP guidelines. • Material: Tube, bell, and blade must be made of 100% stainless steel. |
| Dimensions & Weight | • Total Length: 1100 mm – 1150 mm • Tube/Shaft Length: 700 mm – 750 mm • Diameter: 120 mm – 130 mm. • Net Weight: 6.5 kg – 7.5 kg |
| Ergonomics & Safety | • Handle: Ergonomic handle designed to reduce user fatigue during intensive use . • Power Cord: System must feature a detachable power cord (e.g., "EasyPlug" or equivalent) for easy replacement and safety. • Safety Indicator: Must include a light indicator (e.g., green light) to detect proper cord connection or power cord failure. |
| Necessary Accessories (Included) | To ensure operator safety and ease of use with heavy loads, the offer must include: • Stainless Steel Pan Support: Adjustable or universal mixing tool support that attaches to the rim of the cooking pan, allowing the mixer to be tilted effortlessly or operate autonomously. • Support must be suitable for large pans (diameter range coverage from approx. 500 mm to 1000 mm or universal fit) |
| Documentation | To be supplied in Latvian or English: user manual, electrical diagram, and maintenance instructions. |
| Compliance | Equipment must comply with EU Directives 2006/42/EC, 2014/35/EU, 2014/30/EU, and 1935/2004/EC (food contact). CE marking is mandatory. |
| Guarantee | Minimum 24 months full manufacturer or supplier warranty. |

Name, surname, position

signature