Annex No. 2

TECHNICAL SPECIFICATION

PURCHASE OF HEAVY-DUTY HOOD TYPE UTENSIL WASHERS WITH TABLES

place, date

The following table:

| Parameter | Requirement |
| --- | --- |
| THE PARTICIPANT shall provide information or proof against each requirement. |  |
| Quantity | 4 pcs. – Heavy-duty universal hood-type dishwasher for pots, pans, GN containers, and utensils, complete with inlet and outlet tables. |
| Purpose | Industrial washing of large items (pots, baking sheets, transport baskets, GN 2/1) in a high-throughput commercial kitchen. Machine must be suitable for heavy loads and intensive use. |
| Type | Hood-type / Pass-through utensil washer with oscillating wash arms and high entry height. |
| Capacity & Performance | • Rack Capacity: 25 – 35 racks/hour (theoretical). • Programme Cycles: Minimum 3 adjustable cycles (range 120 / 240 / 480 seconds). • Rack Dimensions: Large washing area, approx. 1280 x 670 mm – 1320 x 700 mm (Stainless steel rack). • Entry Height: Minimum 640 mm – 660 mm to accommodate large pots and crates. |
| Construction | • Material: Double-walled stainless steel construction (AISI 304) for noise and heat insulation. • Protection Class: IP X5. • Wash System: Two powerful wash pumps (total power approx. 4.0 – 4.5 kW) ensuring intensive cleaning from top and bottom. |
| Tank & Consumption | • Tank Filling Volume: 90 – 110 litres. • Rinse Water Consumption: 5.5 – 6.5 litres per cycle to ensure hygiene and efficiency. |
| Electrical Connection | • Total Connected Load: 21.0 kW – 24.0 kW (Target: ~22.9 kW). • Voltage: 400V / 3N PE / 50 Hz. |
| Dimensions (Machine only) | • Width: 1450 mm – 1600 mm. • Depth: 950 mm – 1100 mm. • Height: 1750 mm – 1900 mm (closed hood). |
| Control System | • Interface: Electronic control with digital temperature display for wash and rinse tanks. • Features: Soft start function, self-cleaning programme, leakage detection, infrared interface for data transfer. |
| Mandatory Tables (Original) | The machine must be supplied with original manufacturer's tables designed specifically for this model to ensure seamless integration, stability, and hygiene: 1. Inlet / Pre-wash Table: - Length: 1100 mm – 1300 mm. - Equipment: Deep sink (minimum 500x400x250mm), pre-rinse spray unit (mixer tap with spray hose), splash guard, grid shelf underneath. 2. Outlet / Discharge Table: - Length: 1100 mm – 1300 mm. - Design: Roller or slide surface suitable for heavy racks, end limit switch (if applicable), grid shelf underneath. Total Installation Length: 3800 mm – 4200 mm (Machine + Tables). |
| Complete Functional Set (Mandatory) | Supplier Responsibility: The supplier is fully responsible for providing a complete and operational set for each of the 4 units. The offer must include: • Dosing pumps for detergent and rinse aid (with suction lances). • Drain pump and booster pump (if required for pressure). • Connection kits (water, waste, electric). • 1 x Stainless steel basic rack per machine. • Installation and Commissioning: Assembly of tables and machine, connection to utilities, calibration, and staff training. |
| Documentation | To be supplied in Latvian or English: user manual, electric diagram, installation plan, CE Declaration of Conformity. |
| Compliance | Equipment must comply with EU Machinery Directive 2006/42/EC and DIN 10512 / EN 1717 standards (air gap). CE marking mandatory. |
| Guarantee | Minimum 24 months full manufacturer or supplier warranty. |

Name, surname, position

signature