Annex No. 2

TECHNICAL SPECIFICATION

PURCHASE OF HIGH-CAPACITY VEGETABLE PREPARATION WORKSTATION

place, date

The following table:

| Parameter | Requirement |
| --- | --- |
| THE PARTICIPANT shall provide information or proof against each requirement. |  |
| Quantity | 2 pcs. – Complete vegetable preparation workstation with 3 feed heads, 2 trolleys, and full accessory kit. |
| Brand, model offered | Brand name (for identification purposes) and model name (for identification purposes). |
| Purpose | High-volume processing (slicing, grating, dicing, shredding, french fries, and mashing) of fresh vegetables, fruit, and cheese in a professional catering environment. Designed to process not less then 600 kg of vegetables per hour. |
| Motor Unit | • Type: Industrial induction motor for heavy-duty use. • Construction: Stainless steel motor base. • Power: 1300 W – 1600 W. • Voltage: 400V / 50 Hz / 3-phase. • Speeds: Dual speed (375 and 750 rpm) or Variable Speed ( 100 to 1000 rpm) to adapt to different texture requirements. |
| Feed Heads (3 Included) | The workstation must include three interchangeable feed heads to ensure versatility: 1. Automatic Feed Head: Stainless steel, for continuous feeding of bulk vegetables (potatoes, onions, tomatoes). 2. Pusher Feed Head: Stainless steel, extra-large "Full Moon" hopper (area approx. 220-240 cm² or capacity 4.9 L) for bulky vegetables like cabbage or celeriac. Must include an integrated smaller tube (Ø 58mm) for long vegetables. 3. 4-Tube Feed Head: Designed for long vegetables (cucumbers, leeks). Must feature 4 tubes (e.g., 2x Ø 50mm and 2x Ø 70mm). |
| Trolleys (2 Included) | The workstation must be supplied with two specific trolleys: 1. Ergo Mobile Trolley: Stainless steel trolley to hold the machine and GN pans. Must be adjustable to 3 different heights for transport, loading, and reception. Must be delivered with 3 x GN 1/1 gastronorm containers. 2. Workstation Storage Trolley: Stainless steel trolley designed to store and transport all attachments (heads) and cutting tools (discs). Must include a tray/container for kitchenware. |
| Mashed Potato Equipment | Must include specific equipment for mashing fresh potatoes, consisting of a special feed paddle and a 3mm grid. |
| Cutting Discs (Full Set) | Must include a MultiCut Pack or equivalent comprehensive set of 16 discs covering all major functions: • Slicers: Range from 1mm to 10mm (e.g., 1, 2, 4 mm). • Graters: Range from 1.5mm to 9mm (e.g., 1.5, 3 mm, Parmesan). • Julienne: Various sizes (e.g., 2x10mm, 2.5x2.5mm, 4x4mm). • Dicing Equipment: Grid + Slicer combination (e.g., 10x10x10mm, 20x20x20mm). • French Fries Equipment: Grid + Slicer combination (e.g., 10x10mm). All discs must be "Mineral+" treated or stainless steel for high resistance and dishwasher safety. |
| Hygiene & Safety | • All parts in contact with food must be easily removable and dishwasher safe. • Motor brake and magnetic safety system. |
| Dimensions | • Width: 450 – 480 mm. • Depth: 750 – 800 mm. • Height: 1350 – 1450 mm. • Total Weight: 110 – 130 kg (Net) / 150 – 170 kg (Gross with accessories). |
| Complete Functional Set (Mandatory) | The supplier is responsible for providing a fully functional unit with all listed components: • Motor base + Mobile Ergo Trolley • 3 Feed Heads (Automatic, Pusher, 4-Tube) • Storage Trolley • Potato Ricer Kit • Complete set of 16 discs (MultiCut Pack) • 3x GN 1/1 pans for the trolley |
| Documentation | To be supplied in Latvian or English: user manual, electric diagram, maintenance instructions, CE Declaration of Conformity. |
| Compliance | Equipment must comply with EU Machinery Directive 2006/42/EC and food contact materials legislation. CE marking mandatory. |
| Guarantee | Minimum 24 months full manufacturer or supplier warranty. |

Name, surname, position

signature