# Annex No. 2

TECHNICAL SPECIFICATION

HIGH-PRESSURE PROCESSING (HPP) MACHINE

Place, date: ______________________________

| Parameter | Requirement |
| --- | --- |
| Quantity | 1 pc. – Industrial High-Pressure Processing (HPP) unit for packaged food products. |
| Purpose | In-pack high-pressure processing for ready meals, juices, soups, sauces, fish, meat and other sealed food products; for shelf-life extension and pathogen reduction. |
| Process type | Hydrostatic high-pressure treatment using potable water as pressure-transmitting medium; cold pasteurization of sealed packages. |
| Vessel capacity (usable) | Pressure vessel usable capacity must fall within 50–60 L. |
| Operating pressure | Continuous operating pressure up to 6000 bar (580–600 MPa). |
| Pressure intensification system | Integrated hydraulic intensifiers suitable for continuous HPP cycles. Supplier must specify intensifier type and configuration. |
| Cycle time | Complete treatment cycle (pressurization + holding + depressurization) must be within 120–240 seconds, depending on product and load. Supplier must provide cycle table. |
| Throughput | THE PARTICIPANT must provide hourly throughput depending on product density, loading pattern and cycle recipe. |
| Construction | Vessel, yoke/frame and product baskets must be made of stainless steel. External frame must be stainless steel or coated steel suitable for wash-down environments. |
| Safety system | Machine must comply with PED (Pressure Equipment Directive) and include all required safety blocks, interlocks, pressure sensors, overpressure protection and safety guarding. |
| Hygiene & cleaning | Design must allow full draining, water recirculation and sanitation procedures. Surfaces must be easy to clean and corrosion-resistant. |
| Dimensions (L × W × H) | Overall dimensions must fall within the following ranges: |
|  | Length 2600–2900 mm |
|  | Width 1300–1600 mm |
|  | Height 1800–2100 mm |
| Weight | Minimum 8,000 kg. THE PARTICIPANT must declare actual weight. |
| Electrical power | Total installed electrical power must fall within 85–120 kW. |
| Power supply | 400 V, 3~ (3+N), 50 Hz. |
| Water requirements | Potable water supply for pressure medium and cooling; supplier must specify consumption and water quality requirements. |
| Cooling system | Integrated or external cooling system must be provided. Supplier shall specify cooling capacity, type and requirements. |
| Control system | PLC-based control with touchscreen HMI, recipe management, cycle logging, pressure graphs, alarm history. |
| Data logging | Integrated cycle recording: pressure curves, holding time, temperature, batch ID. Export capability via USB or network. |
| Product baskets / loading | Complete set of product carriers/baskets for full usable vessel volume must be included. |
| Installation & commissioning | Supplier must provide installation plan, anchoring requirements, utility connections and commissioning instructions. |
| Documentation | User manual, maintenance manual, electrical/hydraulic diagrams, pressure vessel certificates, PED documentation, CE Declaration of Conformity. Provided in EN or LV. |
| Compliance and CE marking | Must comply with EU Machinery Directive, EMC Directive, Low Voltage Directive, Pressure Equipment Directive (PED). CE marking mandatory. |
| Guarantee | Minimum 24-month full warranty. |
| Supplier responsibility for full functionality | THE PARTICIPANT must offer such configuration and completeness that fully meets the purpose and technical task, including all necessary sub-components for full operation. |

Name, surname, position

signature