## Annex No. 2

## Technical Specification

### Purchase of Industrial Automated Round Crêpe Production Machine

| Parameter | Requirement |
| --- | --- |
| THE PARTICIPANT shall provide information or proof against each requirement. |  |
| Quantity | 1 pc. – Industrial Automatic Crêpe Production Machine, designed for round crêpes. |
| Purpose | High-volume, continuous production of round crêpes with varying diameters. The machine must ensure consistent quality and thickness suitable for professional and laboratory-scale high output. |
| Operational Specifications |  |
| * Crêpe Shape | Must produce round crêpes. |
| * Crêpe Diameter | The diameter must be adjustable according to customer needs, with a minimal range of 100 mm – 550 mm. |
| * Production Output | High hourly output (measured at Ø200mm): 1200 pcs/h – 2300 pcs/h. |
| * Batter Container Capacity | Equipped with a batter container with a capacity of 48 L – 52 L. |
| * Batter Feeding | The product must be fed to the machine via a dosing pump. |
| * Cooking Temperature | Temperature must be adjustable, up to a maximum of 340C – 360C. |
| * Speed Control | The cooking roller must feature a speed variator controlled by an inverter for adjusting the production speed. |
| * Ventilation | The machine must include an integrated suction hood for steam expulsion |
| Construction & Dimensions |  |
| * Material | Constructed primarily with stainless steel and high-grade aluminum (e.g., EN AW 6082 T6 or equivalent) |
| * Electrical Panel | Electrical panel must be safely mounted on board the machine, built in respect of all accident prevention regulations in force |
| * Nominal Weight | 370 kg – 390 kg |
| * External Length (A) | 1300 mm – 1320 mm |
| * External Depth (B) | 1180 mm – 1200 mm |
| * External Height (C) | 1370 mm – 1400 mm |
| Energy & Power |  |
| * Max. Installed Power | 27 KW – 30 KW |
| * Supply Voltage | 3x400V, 50/60 Hz (or compatible industrial voltage). |
| Complete Functional Set (Mandatory) | The Supplier must ensure the delivery of a fully operational unit, complete with all necessary components, accessories, and ancillary tools required for continuous crêpe production. This explicitly includes: |
|  | • The dosing pump and all transfer mechanisms. |
|  | • The inverter-controlled speed variator. |
|  | • The integrated suction hood. |
|  | • All necessary tools for operation, calibration, and cleaning. |
|  | • Installation materials, final utility connections, commissioning, and comprehensive staff training. |
| Documentation | To be supplied in Latvian or English: user manual, electric diagram, maintenance instructions, and CE Declaration of Conformity. |
| Guarantee | Minimum 24 months full manufacturer or supplier warranty. |