## Annex No. 2
Technical Specification

### Purchase of Industrial Automatic Pasta Extruder with Accessories

| Parameter | Requirement |
| --- | --- |
| THE PARTICIPANT shall provide information or proof against each requirement. |  |
| Quantity | 1 pc. – Industrial Automatic Pasta Extruder (Tabletop Unit). |
| Purpose | Automatic and reliable production of both short and long pasta shapes, as well as specialized shapes (ravioli and gnocchi), processing all common types of flour and semolina. Suitable for continuous medium-volume production (e.g., restaurants, delicatessens). |
| Main Machine Specifications |  |
| * Kneading Vat Capacity | Max. capacity: 2.5 kg – 3.5 kg per batch. |
| * Extrusion Production | Output capacity: 8 kg/h – 11 kg/h. |
| * Automatic Cutter | Includes an integrated Automatic Pasta Cutter with a speed variator for cutting short pasta shapes. |
| * Fan | Includes an integrated pre-drying fan. |
| * Construction | Exterior structure made of anodized aluminum; all parts in contact with pasta (shaft, auger, tank interior) must be Stainless Steel (AISI 304 or equivalent). |
| * Detachable Parts | Shaft and auger must be detachable for easy cleaning and maintenance. |
| * Dimensions (W x D x H) | Max. Dimensions: 400 mm – 510 mm (W) x 360 mm – 510 mm (D) x 440 mm – 470 mm (H). |
| * Nominal Weight | 55 kg – 65 kg. |
| * Power / Voltage | Power: 0.5 kW – 1.0 kW. Voltage: 220V/1Ph/50Hz (Single-phase) or 380V/3Ph/50Hz (Three-phase) as specified by Procuring Authority. |
| Mandatory Accessory 1: Ravioli Unit |  |
| * Function | Must be a dedicated attachment for the production of ribbon ravioli in various sizes with soft fillings (vegetables, cheese, meats). |
| * Capacity | Production capacity: 17 kg/h – 21 kg/h. |
| * Unit Weight | 17 kg – 19 kg. |
| * Installation | Must be designed for simple and quick attachment to the main machine. |
| Mandatory Accessory 2: Gnocchi Unit |  |
| * Function | Must be a dedicated attachment for the production of potato gnocchi or gnocchi made from various flours. |
| * Adaptability | The shape of the gnocchi must be easily changed by replacing the forming tool. |
| * Installation | Must be designed for simple and practical attachment to the main machine. |
| Mandatory Accessory 3: Complete Die Set |  |
| * Coverage | The offer must include a comprehensive set of all available pasta dies designed for this machine, covering all short pasta, long pasta, and pasta sheet formats. |
| * Die Material | The supplied dies must include both main material types available for the machine: Metal (Bronze/Brass) dies (for rough pasta surface) AND PTFE Insert dies (for smoother, shinier pasta surface). |
| Complete Functional Set (Mandatory) | The Supplier must fully complete the equipment, understanding the intended purpose (comprehensive fresh pasta production) and including all necessary components and required accessories. This explicitly includes: |
|  | • The main extruder unit. |
|  | • The Ravioli Unit (Mandatory Accessory 1). |
|  | • The Gnocchi Unit (Mandatory Accessory 2). |
|  | • The Comprehensive Die Set (Mandatory Accessory 3). |
|  | • All necessary frames, brackets, filling systems (for Ravioli), and pasta trays. |
|  | • Installation materials, utility connections, commissioning, and comprehensive staff training. |
| Documentation | To be supplied in Latvian or English: user manual, electric diagram, maintenance instructions, and CE Declaration of Conformity. |
| Guarantee | Minimum 24 months full manufacturer or supplier warranty. |