## Annex No. 2

## Technical Specification

### Purchase of Industrial Baguette Moulder (Three-Cylinder Type)

| Parameter | Requirement |
| --- | --- |
| THE PARTICIPANT shall provide information or proof against each requirement. |  |
| Quantity | 1 pcs. – Industrial Baguette Moulder (Three-Cylinder Configuration). |
| Purpose | To form long bread products, such as baguettes, long loaves (filoni), and similar shapes. The machine must laminate, wrap, roll, and extend the dough pieces accurately. |
| External Dimensions (Single Unit) |  |
| * Total External Width (A) | 1040 mm – 1060 mm. |
| * Total External Depth (B) | 940 mm – 960 mm. |
| * Total External Height (H) | 1490 mm – 1520 mm. |
| Operational Specifications |  |
| * Dough Weight Range (Min) | Minimum dough piece weight: 90 g – 110 g. |
| * Dough Weight Range (Max) | Maximum dough piece weight: 1200 g – 1300 g. |
| * Working Width | Belt cylinder/carpet length: 700 mm – 710 mm. |
| * Forming Process | Dough is first laminated, then wrapped, rolled, and extended using two belts rotating in opposite directions. |
| * Rolling Cylinders | Rolling/Laminating cylinders must be made of non-toxic material (e.g., food-safe polyethylene). |
| * Adjustment | Must allow for precise and independent regulation of the rolling cylinders and the extension belts according to the dough portion weight. |
| Technical & Energy |  |
| * Motor Power | 0.50 kW – 0.60 kW. |
| * Weight | 230 kg – 250 kg. |
| * Mobility | Must be designed for mobility, mounted on a basement equipped with wheels. |
| * Safety | Must include a safety bar or equivalent emergency stop mechanism. |
| Compliance & Documentation |  |
| * Certificates | Equipment must comply with CE rules (or equivalent EU Directives) and hold EAC certification. |
| Complete Functional Set (Mandatory) | The Supplier must ensure the delivery of a fully functional machine, ready for the specified task. The offer must explicitly include: all necessary components, belts, cylinders, installation materials, commissioning, and comprehensive staff training. The Supplier is responsible for coordinating with the Procuring Authority to ensure the equipment performs its function as requested. |
| Guarantee | Minimum 24 months full manufacturer or supplier warranty. |