## Annex No. 2

## Technical Specification

### Purchase of Industrial Blast Chiller / Shock Freezer

| Parameter | Requirement |
| --- | --- |
| THE PARTICIPANT shall provide information or proof against each requirement. |  |
| Quantity | 1 pc. – Industrial Blast Chiller / Shock Freezer (Roll-In Cabinet Type). |
| Purpose | To ensure rapid chilling (blast chilling) and shock freezing (shock freezing) of cooked food products immediately after production to stop bacterial proliferation, preserve organoleptic properties, and ensure HACCP compliance. Suitable for large catering operations (Cook & Chill system). |
| External Dimensions (Max.) |  |
| * Total External Width (Length) | 1350 mm – 1360 mm. |
| * Total External Depth (Closed Door) | 1230 mm – 1240 mm. |
| * Total External Height | 2190 mm – 2200 mm. |
| Internal Capacity and Dimensions |  |
| * Rack Capacity | Must accommodate 20 trays of GN 1/1 or 600x400 mm size (max. tray height 65 mm). |
| * Max. Trolley Dimensions | Maximum trolley dimensions: 720 mm (Width) x 900 mm (Depth) x 1880 mm (Height). |
| Operational Cycles (Production Capacity) |  |
| * Positive Cycle (Chilling) | From +90C to +3C (in less than 90 min.). |
| * Chilling Capacity (Max.) | 85 kg – 100 kg per cycle. |
| * Negative Cycle (Freezing) | From +90 to -18C (in less than 240 min.). |
| * Freezing Capacity (Max.) | 60 kg – 70 kg per cycle. |
| Technology and Control |  |
| * Control Panel | Electronic LCD board (LCD Electronic Board) capable of automatic cycle calculation ("Chilly" system or equivalent). |
| * Main Cycles | Positive (Normal/Soft, Intensive/Hard) and Negative (Normal/Soft, Intensive/Hard). |
| * Special Functions | Mandatory cycles include: Fish Sanitization (Sanificazione Pesce), Ice Cream Hardening (Indurimento Gelato), Slow Cooking (Lenta Cottura), Thawing (Scongelamento), and Proofing (Lievitazione). |
| * Probe Control | Mandatory heated core probe as standard equipment. |
| Construction and Hygiene |  |
| * Material | Monobloc structure entirely built with AISI 304 Stainless Steel. |
| * Insulation | 80 mm thickness insulation with high density polyurethane foam. |
| * Hygiene | Chambers must have rounded internal corners for easier cleaning. |
| * Access | Easy accessibility to the evaporator for cleaning. |
| * Door | Must be equipped with a removable magnetic seal and micro door sensor to stop the fan when the door is open. |
| Refrigeration and Energy |  |
| * Refrigerant Type | R452A (GWP 2141) or equivalent modern refrigerant with low GWP. |
| * Defrost | Automatic defrosting by hot gas. |
| * Supply Voltage | 3x400V, 50/60Hz (or equivalent 3-phase industrial voltage). |
| * Internal Unit Power (Max.) | Up to 550 W. |
| Complete Functional Set (Mandatory) | The Supplier must ensure the delivery of a fully operational unit, complete with the heat pump, control panel, heated core probe, all motors, fans, seals, and connecting materials. The offer must explicitly include: installation materials, commissioning, and comprehensive staff training. |
| Documentation | To be supplied in Latvian or English: user manual, electric diagram, maintenance instructions, and CE Declaration of Conformity. |
| Guarantee | Minimum 24 months full manufacturer or supplier warranty. |