## Annex No. 2

## Technical Specification

### Purchase of Industrial Cold System Pizza Dough Forming Machines

| Parameter | Requirement |
| --- | --- |
| THE PARTICIPANT shall provide information or proof against each requirement. |  |
| Quantity | 2 pcs. – Industrial Cold System Pizza Dough Forming Machines (Spinners). |
| Purpose | To efficiently flatten and spread pizza dough portions without altering the dough's temperature or elasticity. The process must closely replicate the manual skills of an expert pizza maker, achieving an artisanal result. |
| External Dimensions (Single Unit) |  |
| * Total External Width (A) | 500 mm – 520 mm. |
| * Total External Depth (B) | 720 mm – 740 mm. |
| * Total External Height (H) | 760 mm – 780 mm. |
| Operational Specifications |  |
| * Working System | Must utilize a Cold System (non-heating) for processing dough. |
| * Core Technology | Must employ a Patented Micro-Rolling System (or equivalent protected technology) to ensure gentle dough handling. |
| * Final Product Feature | Must be capable of achieving the pizza edge/crust execution (rim) typical of hand-made pizza. |
| * Adjustable Thickness | The machine must allow for adjustable pizza thickness. |
| * Maximum Pizza Diameter | Max. finished pizza diameter (with crust): 440 mm – 460 mm. |
| * Production Capacity | High hourly capacity: 200 – 250 pieces/hour. |
| * Operation | Must feature Automatic Start and Stop functionality. |
| Technical & Energy |  |
| * Motor Power | 0.70 kW – 0.80 kW (for 220V/1/50Hz). |
| * Weight | 110 kg – 120 kg. |
| * Voltage / Frequency | 220V / 1-Phase / 50 Hz or as per technical requirement (Special Tensions optional). |
| * Noise Level | Low noise level: < 70 dB(A). |
| * Protection Degree | Motor and electrical devices must meet at least IP55 standard. |
| * Construction Material | Standard construction material: Industrial grade exterior (Stainless Steel version optional). |
| * Maintenance | Designed for low energy consumption and low maintenance costs |
| Compliance & Documentation |  |
| * Certificates | Equipment must comply with CE rules (or equivalent EU Directives) and hold EAC certification. |
| Complete Functional Set (Mandatory) | The Supplier must ensure the delivery of two fully functional units ready for the specified task. The offer must explicitly include: installation materials, commissioning, and comprehensive staff training. |
| Guarantee | Minimum 24 months full manufacturer or supplier warranty. |