## Annex No. 2

## Technical Specification

### Purchase of Industrial Electric Bath Pasteurizers for Bottles

| Parameter | Requirement |
| --- | --- |
| THE PARTICIPANT shall provide information or proof against each requirement. |  |
| Quantity | 2 pcs. – Industrial Bath Pasteurizers, designed for bottles, jars, or similar sealed containers. |
| Purpose | To achieve precise and controlled pasteurization of filled and sealed products using an immersion system (water bath), ensuring food safety and extending shelf life. Designed for semi-automatic batch processing. |
| Operational Specifications |  |
| * Nominal Output Capacity (Per Unit) | 190 bottles/h – 220 bottles/h (Based on 1.0 L volume or equivalent). |
| * Heating Source | Electric. |
| * Max. Heating Temperature | Capable of reaching and maintaining a maximum water temperature of 88C – 95C. |
| * Process | System must be designed for bottles/jars to be placed into trays, which are then loaded into the bath. The water level must reach the bottle neck before the process begins. |
| * Control | Integrated control panel for setting the desired pasteurization temperature and holding time. |
| * Water Management | Must feature an outlet or mechanism for releasing the water from the bath at the end of the timer cycle. |
| External Dimensions (Per Unit) |  |
| * Total External Length (L) | 1470 mm – 1490 mm. |
| * Total External Width (W) | 860 mm – 880 mm. |
| * Total External Height (H) | 920 mm – 940 mm. |
| Technical & Energy |  |
| * Nominal Weight | 110 kg – 120 kg (empty). |
| * Supply Voltage | 3x400V, 50 Hz (or compatible industrial voltage). |
| * Max. Connected Load | 10 kW – 16 kW. |
| Construction & Hygiene |  |
| * Material | Entire structure and bath tub must be made of Stainless Steel (AISI 304 or equivalent). |
| * Tray System | Must include a system of loadable trays compatible with the bath size (for faster exchange process). |
| Complete Functional Set (Mandatory) | The Supplier must ensure the delivery of two fully functional units, complete with all necessary heating elements, control panel, pumps, and safety systems. The offer must explicitly include: |
|  | • All required internal components. |
|  | • Necessary trays/racks for holding the bottles during the process. |
|  | • Installation materials and final utility connections. |
|  | • Commissioning and comprehensive staff training on proper pasteurization process and core temperature monitoring. |
| Documentation | To be supplied in Latvian or English: user manual, electric diagram, maintenance instructions, and CE Declaration of Conformity. |
| Guarantee | Minimum 24 months full manufacturer or supplier warranty. |