## Annex No. 2

## Technical Specification

### Purchase of Industrial Electronic Water Dosing and Mixing Unit

| Parameter | Requirement |
| --- | --- |
| THE PARTICIPANT shall provide information or proof against each requirement. |  |
| Quantity | 1 pc. – Industrial Electronic Water Dosing and Mixing Unit. |
| Purpose | To automatically dose and mix warm, refrigerated, and tap water (pagal instaliacijos tipą) and deliver the precise quantity and temperature of water required into a dough mixer. This ensures accurate control over the dough temperature, critical for baking consistency. |
| System & Control |  |
| * Dosing Mechanism | Electronic dosing-unit equipped with a microprocessor. |
| * Interface | Digital touch-type keyboard for setting quantity and temperature. |
| * Recipe Capacity | Must be programmable for a minimum of 80 different recipes. |
| * Maximum Dosing Capacity | Up to 999.9 litres (min. 999.0 L- max. 1000.0 L). |
| * Resolution | Must have a dosing resolution of tenth of a litre (0.1 L). |
| Monitoring & Accuracy |  |
| * Displays | Equipped with minimum 3 displays for simultaneous visualization of: water quantity, water temperature, and temperature controlled by an external probe. |
| * External Probe | Includes a unit with an external probe to measure either the environment/external temperature or the mixing temperature. |
| * Temperature Control | Must automatically dose and mix the water sources to reach the set target temperature. |
| * Water Draining | Must include a ball valve for draining water that is not at the correct temperature. This drainage can be controlled thermostatically or by time. |
| Construction & Integration |  |
| * Material | Stainless steel structure. |
| * Water Source Mixing | Must be capable of mixing multiple water sources: warm, refrigerated, and pipe (tap) water. |
| Technical & Energy |  |
| * Power Supply | 220V/400V (to be specified at final order) / 50 Hz. |
| Complete Functional Set (Mandatory) | The Supplier must ensure the delivery of a fully integrated and functional unit, complete with all sensors, valves, connections (for three water sources), control panel, and probe. The offer must explicitly include: installation materials, commissioning, and comprehensive staff training. |
| Documentation | To be supplied in Latvian or English: user manual, electric diagram, maintenance instructions, and CE Declaration of Conformity. |
| Guarantee | Minimum 24 months full manufacturer or supplier warranty. |