Annex No. 2

TECHNICAL SPECIFICATION

PURCHASE OF INDUSTRIAL FRUIT AND VEGETABLE WASHING MACHINE (HELICAL SYSTEM)

place, date

The following table:

| Parameter | Requirement |
| --- | --- |
| THE PARTICIPANT shall provide information or proof against each requirement. |  |
| Quantity | 1 pc. – Continuous washing machine for fruits, vegetables, and salads with vibration outfeed. |
| Brand, model offered | Brand name (for identification purposes) and model name (for identification purposes). |
| Purpose | Continuous pre-washing, washing, disinfection, mixing, and treatment of whole and cut lettuce, vegetables, herbs, and fruit. Suitable for baby leaf, iceberg, spinach, tuber products, pulses, and mushrooms. |
| Washing System | • Type: HELICAL washing system ensuring a spiral water movement for thorough and gentle cleaning. • Flow Control: Adjustable water flow rate and movement to suit different products (floating or sinking). • Sand Separation: Integrated sand trap sedimentation unit in the wash tank to separate heavy dirt (sand, small stones) from the product flow. |
| Capacity | • Processing Capacity: Up to 550 – 650 kg/h, depending on the product. |
| Construction | • Material: Entire construction (tank, piping, pump casing, covers, switch cabinet) made of stainless steel AISI 304. • Design: Compact, modular design allowing for flexible adaptation and easy cleaning/maintenance (hygienic design). |
| Tank Volume | • Total Water Volume: 500 – 550 litres |
| Outfeed System | • Type: Vibration outfeed mechanism. • Function: Ensures preliminary dewatering of the product and sorting out of small particles. • Rinsing: Integrated fresh water shower above the outfeed to rinse off residues or disinfectant. |
| Water Management | • Fresh Water Connection: 3/4 inch (GEKA coupling or equivalent). • Drainage: 2 x 2" drain outlets. • Recirculation: Pump system to recirculate water from the pump tank back to the wash tank through a filter strainer. |
| Dimensions | • Length: 2700 – 2800 mm. • Width: 950 – 1050 mm. • Height: 1450 – 1550 mm. • Infeed Height: 1050 – 1150 mm. • Outfeed Height: 750 – 800 mm. |
| Weight | • Net Weight: 370 – 400 kg. |
| Electrical Connection | • Power: 3.0 – 3.5 kW. • Voltage: 400V / 3 ph / 50 Hz (N/PE). |
| Complete Functional Set (Mandatory) | The supplier is responsible for providing a fully functional unit with all necessary components for immediate operation, including: • Complete machine body with wash tank and vibration deck • Integrated switch cabinet/control panel • All necessary internal piping and pumps • Filter strainers for the vibration deck (e.g., 2, 5, 8, or 10mm hole size – to be specified or standard included) • Fresh water rinsing bar • Sand trap system • Levelling feet |
| Documentation | To be supplied in Latvian or English: user manual, electric diagram, maintenance instructions, CE Declaration of Conformity. |
| Compliance | Equipment must comply with EU Machinery Directive 2006/42/EC and food contact materials legislation. CE marking mandatory. |
| Guarantee | Minimum 24 months full manufacturer or supplier warranty. |

Name, surname, position

signature