## Annex No. 2

## Technical Specification

### Purchase of Industrial High-Capacity Proofer/Proofing Chambers

| Parameter | Requirement |
| --- | --- |
| THE PARTICIPANT shall provide information or proof against each requirement. |  |
| Quantity | 2 pcs. – Industrial Proofing Chambers, designed for high-capacity, controlled environment leavening of dough products. |
| Purpose | To store dough products for baking in an environment with precisely controlled temperature and humidity, guaranteeing optimal and consistent leavening results prior to transfer into the oven. |
| Operational Specifications |  |
| * Compatibility | Must be designed to integrate and be fully compatible with the rotating trolleys/racks specified for the high-capacity convection ovens (compatible with 600 mm x 800 mm and 400 mm x 600 mm tray sizes). |
| * Trolley Capacity (per unit) | Capable of accommodating a minimum of 3 – 4 rotating trolleys (racks) simultaneously. |
| * Max. Internal Temperature | Maximum achievable temperature: 45C – 50C. |
| * Max. Internal Humidity | Maximum achievable relative humidity: 85% – 90%. |
| External Dimensions (Single Unit) |  |
| * Total External Width (A) | 210 cm – 225 cm. |
| * Total External Depth (B) | 170 cm – 180 cm. |
| * Total External Height (H) | 240 cm – 250 cm. |
| Energy & Heating (Single Unit) |  |
| * Power Source | Electric. |
| * Total Connected Load | 4.0 kW – 5.0 kW. |
| * Voltage | 400V / 3N PE / 50 Hz or equivalent industrial standard. |
| Construction & Features |  |
| * Material | Internal and external structure must be made of Stainless Steel (Inox). |
| * Door Configuration | Must feature 2 doors. The required side for the door opening must be specified by the Procuring Authority during the ordering phase |
| * Insulation | Double-walled or equivalent high-quality insulation to maintain temperature and humidity stability. |
| Control System and Automation |  |
| * Control Panel | Electronic control panel accessible from the exterior. |
| * Functions | Must allow setting the desired internal temperature (up to max. 50C) and relative humidity (up to max. 90%). |
| * Steam Extraction | Must be equipped with a functional system for steam control and extraction. Steam Extractor Flow Rate: 1 – 6 m3/min. |
| Mandatory Accessories & Completeness |  |
| * Compatibility Feature | The chamber must be designed to allow for the direct transfer of the trolley from the chamber into the rotating oven, eliminating the need to shift trays and saving time/labour. |
| Complete Functional Set (Mandatory) | The Supplier shall ensure the delivery of two fully functional proofing chambers, complete with all necessary sensors, humidifiers, and control components, ready for immediate operation. The offer must explicitly include: |
|  | • All required internal components. |
|  | • Installation materials and final connection to utilities (e.g., water, electricity). |
|  | • Commissioning and comprehensive staff training. |
| Documentation | To be supplied in Latvian or English: user manual, electric diagram, maintenance instructions, and CE Declaration of Conformity. |
| Compliance | Equipment must comply with relevant EU directives (e.g., Machinery Directive 2006/42/EC) and food hygiene standards. CE marking is mandatory. |
| Guarantee | Minimum 24 months full manufacturer or supplier warranty. |