## Annex No. 2

## Technical Specification

### Purchase of Industrial Mechanical Dough Sheeter (Mobile Base Model)

| Parameter | Requirement |
| --- | --- |
| THE PARTICIPANT shall provide information or proof against each requirement. |  |
| Quantity | 1 pc. – Industrial Mechanical Dough Sheeter (Mobile Base Model or equivalent). |
| Purpose | Precise and gentle sheeting of almost all types of dough (e.g., puff pastry, yeast dough, shortcrust) into consistent dough blocks and bands. Suitable for bakeries, hotels, restaurants, pizza shops, or canteen kitchens. |
| Model Type | Mobile base model with fold-up tables. |
| Capacity / Performance |  |
| * Recommended Operation | Designed for 1-shift operation. |
| * Capacity (Dough per hour) | 20 kg/h – 50 kg/h. |
| * Dough Block Size | Suitable for dough blocks up to 3 kg – 4 kg. |
| * Sheeting Speed | Discharge conveyor speed: 50 cm/s. |
| Technical Parameters |  |
| * Conveyor Belt Width | 475 mm – 480 mm. |
| * Roller Length | 500 mm – 510 mm. |
| * Roller Gap | Adjustable from 0.3 mm – 30 mm. |
| * Roller Diameter | 84 mm – 85 mm. |
| * Roller Gap Reduction | Manual adjustment. |
| * Substructure | Sturdy A-frame substructure, preferably made of stainless steel. |
| * Transport | Easy to move and transport. |
| Dimensions & Space Requirement |  |
| * Table Length Overall | 2050 mm – 2060 mm. |
| * Required Floor Space (Working) | 2500 mm – 2550 mm (Length x Width). |
| * Required Floor Space (Resting) | 1100 mm – 1150 mm (Length x Width). |
| * Weight (Mobile Base Model) | 140 kg – 150 kg. |
| Energy & Controls |  |
| * Rated Power | 0.75 kVA / 0.5 kW. |
| * Supply Voltage | 3x400 V, 50/60 Hz (or compatible 3-phase industrial voltage). |
| * Operation** | Operating elements and controls must be easy to reach. Features an ergonomically shaped roller adjustment lever. |
| Mandatory Accessories (Complete Set) | The price proposal must include all applicable accessories designed to adapt and maximize the functionality of the dough sheeter: |
|  | 1. Flour Catch Pans: To collect excess flour, reducing cleaning effort and slipping risk. |
|  | 2. Dough Catch Pans: Fitted at the conveyor belt ends to catch excess dough. |
| Complete Functional Set (Mandatory) | The Supplier must ensure the delivery of a fully integrated and functional dough sheeter unit, complete with the main unit, mobile base, fold-up tables, and all two specified mandatory accessories. The offer must explicitly include: installation materials, commissioning, and comprehensive staff training. |
| Documentation | To be supplied in Latvian or English: user manual, electric diagram, maintenance instructions, and CE Declaration of Conformity. |
| Guarantee | Minimum 24 months full manufacturer or supplier warranty. |