## Annex No. 2

## Technical Specification

### Purchase of Industrial Refrigerated Preparation Counters (Work Stations)

| Parameter | Requirement |
| --- | --- |
| THE PARTICIPANT shall provide information or proof against each requirement. |  |
| Quantity | 2 pcs. – Industrial Refrigerated Work Stations, specifically designed for intensive pizza or pastry preparation applications. |
| Purpose | To provide a refrigerated storage base and an integrated preparation area for high-volume tasks, ensuring instant access to fresh ingredients and optimal food preservation during preparation time. |
| External Dimensions (Single Unit) |  |
| * Total External Width | 2000 mm – 2030 mm (Width). |
| * Total External Depth | 800 mm – 815 mm (Depth). |
| * Total External Height (Base) | 850 mm – 900 mm (Height of the counter top, adjustable legs not included in this range). |
| Internal Volume | Total Volume: 670 L – 700 L. |
| Internal Configuration | The counter must be compatible with EN 60x40 or GN 1/1 internal dimensions (drawers and grids). |
| Refrigeration & Environment |  |
| * Working Temperature (Storage) | 0C – +8C. |
| * Worktop Refrigeration (Vitrines) | 0C – +8C or lower, ensuring cold air circulation from the main unit. |
| * Defrost System | Automatic Defrost (Stop cycle). |
| * Cooling System | Forced air cooling system (Ventilated). |
| * Climate Class | Must be rated for a minimum of Climate Class 4 (suitable for 30C and 55% humidity). |
| Worktop & Ingredients Station |  |
| * Worktop Material | Heavy-duty Stainless Steel working top with 50 mm insulation. |
| * Integrated Ingredient Unit | Built-in neutral or refrigerated compartment in the worktop designed to house ingredients containers. |
| * Ingredient Capacity (Containers) | Must accommodate: 9 – 10 GN 1/3 containers (max. 100 mm depth) OR 16 – 17 GN 1/4 containers (max. 100 mm depth). |
| * Lid | Must include a Stainless Steel hinged lid for the ingredient area, with a fixable 90 degrees opening position. |
| Construction & Hygiene |  |
| * Material | Interior constructed with AISI-304 stainless steel. Exterior constructed with food-grade stainless steel. |
| * Doors | Self-closing hinged doors. |
| * Interior Design | Smooth surfaces and internal base with rounded corners for easy cleaning. |
| * Drain | Includes a built-in drain for waste water evacuation. |
| * Insulation | Minimum insulation thickness: 50 mm. |
| Technical & Accessories |  |
| * Electric Power (Max. Connected Load) | Max. Electric Power: 0.50 kW – 0.55 kW2. |
| * Voltage / Frequency | 220-240V, 50 Hz / 60 Hz compatible. |
| * Standard Accessories (per unit) | The offer must include: 3 pcs of removable and height-adjustable epoxy coated EN 60x40 grids and the corresponding 6 pcs of support guides. |
| * Standard Features | Height-adjustable stainless steel legs (130 mm to 200 mm), LED interior lighting, and removable magnetic door gaskets. |
| Complete Functional Set (Mandatory) | The Supplier must ensure the delivery of two fully functional units ready for the specified task. The offer must explicitly include: installation materials, commissioning, and comprehensive staff training. |
| Documentation | To be supplied in Latvian or English: user manual, electric diagram, maintenance instructions, and CE Declaration of Conformity. |
| Compliance | Equipment must comply with relevant EU directives (e.g., Machinery Directive 2006/42/EC). CE marking is mandatory. |
| Guarantee | Minimum 24 months full manufacturer or supplier warranty. |