## Annex No. 2

## Technical Specification

### Purchase of Industrial Single-Lane Hygiene Stations (Entry/Exit Control)

| Parameter | Requirement |
| --- | --- |
| THE PARTICIPANT shall provide information or proof against each requirement. |  |
| Quantity | 6 pcs. – Industrial Single-Lane Hygiene Stations (Full-cycle control). |
| Purpose | To enforce mandatory hygiene procedures (boot cleaning/sanitization and hand sanitization) for personnel entering or exiting controlled production zones, ensuring optimal personnel hygiene and minimizing contamination risk. |
| Operational Control |  |
| * Access Frequency | Designed for high access frequency, enabling the passage of 10 – 15 persons per minute (approx. 4 – 6 seconds per person). |
| * Access Control | Must integrate a turnstile/tripod gate which only unlocks upon successful completion of the hand sanitization cycle |
| * Operational Direction | Must be suitable for Entry and Exit control. |
| * Orientation | The Supplier must coordinate with the Procuring Authority (Pirkimų skyrius) prior to final manufacturing/delivery to confirm the required Right or Left positioning/orientation of the station. |
| Footwear Cleaning & Sanitization |  |
| * Cleaning Method | Automatic cleaning of footwear soles (pads) and sides. |
| * Brush Type | Must use Vertical brushes (V-type) for sides, su mažiausiu šepetėlių aukščiu 70 mm – 80 mm. |
| * Operation | Automatic activation of brushes ir detergento/dezinfekcinio skysčio dozavimo po įlipimo. |
| * Detergent/Disinfectant | Integrated tanks for holding and automatically dosing cleaning agent and disinfectant. |
| Hand Sanitization |  |
| * Method | Non-contact (sensor-activated) hand sanitization system. |
| * Cycle | Must sanitize both hands simultaneously. |
| * Sanitizer | Suitable for liquid or gel sanitizers; includes a level indicator. |
| * Dosing | Adjustable dosing volume. |
| Construction & Hygiene Standards |  |
| * Material | Entire structure, brushes, and railing must be constructed from AISI 304 Stainless Steel (or equivalent food-grade, high-corrosion-resistance material). |
| * Design | Robust, ergonomic design with smooth surfaces and integrated drainage, ensuring easy cleaning and compliance with HACCP principles. |
| * Water Supply | Connection points for cold/hot water (for cleaning) and drainage required. |
| Technical & Energy |  |
| * Power Supply | 400V / 3N PE / 50 Hz or 230V / 1-Phase / 50 Hz (to be confirmed). |
| * Max. Connected Load | 0.8 kW – 1.0 kW. |
| * Water Inflow | Min. 3/4-inch (or equivalent) connection. |
| Complete Functional Set (Mandatory) | The Supplier must ensure the delivery of six fully functional units, complete with all dosing pumps, sensors, internal plumbing, electric motors, turnstile mechanism, and external connection points. The offer must explicitly include: installation materials, commissioning, and comprehensive staff training. |
| Documentation | To be supplied in Latvian or English: user manual, electric diagram, maintenance instructions, and CE Declaration of Conformity. |
| Guarantee | Minimum 24 months full manufacturer or supplier warranty. |