Annex No. 2

TECHNICAL SPECIFICATION

PURCHASE OF INDUSTRIAL SPIRAL MIXER WITH REMOVABLE BOWL (HEAVY DUTY)

place, date

The following table:

| Parameter | Requirement |
| --- | --- |
| THE PARTICIPANT shall provide information or proof against each requirement. |  |
| Quantity | 1 pcs. – Heavy-duty industrial spiral mixer with removable bowl and full accessory package. |
| Brand, model offered | Brand name (for identification purposes) and model name (for identification purposes). |
| Purpose | Intensive industrial kneading of bread dough, yeast dough, and baking mixtures. Specifically designed for heavy-duty applications, including mixtures with humidity less than 50%. |
| Capacity | • Dough Capacity: 150 – 170 kg. • Flour Capacity: 95 – 110 kg. • Bowl Volume: 260 – 290 litres. |
| Construction (Full Stainless Steel) | • Structure: "Stainless Steel Version" – The entire machine frame/body, not just contact parts, must be made of Stainless Steel AISI 304 for maximum hygiene and durability. • Moving Parts: Bowl, spiral, and shaft must be made of high-strength Stainless Steel. • Lifting System: Hydraulic system with a large-sized piston for opening/closing the head to allow bowl removal. |
| Motors & Power (HD Version) | • Configuration: Machine must have at least 2 independent motors (one for spiral, one for bowl) plus a hydraulic unit. • Spiral Motor Power (High Power Version): 11.0 kW – 16.0 kW (Reinforced motor for stiff doughs). • Bowl Motor Power: 0.75 kW – 1.5 kW. • Voltage: 400V / 3 ph / 50 Hz (Special tension ready if required). |
| Control System (PLC Mode) | • Type: PLC Control Panel (Touchscreen) with software for recipe management. • Speed Control: Equipped with Inverters (VFD) to allow variable control of both Spiral Speed and Bowl Speed. • Automation: Programmable timers, work cycles, and diagnostic capability. |
| Bowl Mechanism | • Rotation: Bowl rotation guaranteed by friction wheels for high grip. • Reversibility: Capable of inverse rotation. • Locking: High-powered electromagnetic system to block the bowl during kneading. • Drainage: Bowl equipped with a Drain Bowl Cap for washing/cleaning. |
| Integrated Dosing & Monitoring | • Water Dosing: Integrated water dosing system controlled by the PLC. • Temperature Probe: Integrated infrared or contact temperature probe to monitor dough temperature on the display. |
| Mixing Tools | • Spiral: Special/Reinforced tool for heavy-duty mixing. • Scraper: Integrated bowl scraper to ensure complete mixing of ingredients. |
| Dimensions & Weight | • Width: 1000 mm – 1150 mm. • Depth: 1900 mm – 2100 mm. • Height (Closed): 1550 mm – 1750 mm. • Net Weight: 1250 kg – 1400 kg |
| Complete Functional Set (Mandatory) | The supplier is fully responsible for providing a complete operational set. The offer must include: • 1 x Main Mixer Unit (Stainless Steel Body, HD Motors) • 1 x Removable Stainless Steel Bowl (Standard) • 1 x EXTRA Removable Stainless Steel Bowl on wheels (Total 2 bowls) • Full set of sensors (Temperature, Safety) • All necessary hydraulic and electrical components. • Installation and commissioning. |
| Documentation | To be supplied in Latvian or English: user manual, electric diagram, maintenance instructions, CE Declaration of Conformity. |
| Compliance | Equipment must comply with EU Machinery Directive 2006/42/EC, EAC, and ETL standards. CE marking mandatory. |
| Guarantee | Minimum 24 months full manufacturer or supplier warranty. |

Name, surname, position

signature