## Annex No. 2

## Technical Specification

### Purchase of Industrial Stainless Steel Standard Containers (Eurobins)

| Parameter | Requirement |
| --- | --- |
| THE PARTICIPANT shall provide information or proof against each requirement. |  |
| Quantity | 10 pcs. – Industrial Stainless Steel Containers (Eurobin standard). |
| Purpose | Standartizuotas transportavimas, laikymas, kėlimas ir apdorojimas maisto pramonėje, ypač mėsos pramonėje. Skirta naudoti su standartine kėlimo ir vartymo įranga. |
| Standard Compliance | Containers must strictly comply with DIN 9797 standard (arba lygiavertis Europos maisto pramonės standartas). |
| Material | All containers must be constructed from high-grade AISI 304 Stainless Steel (or equivalent food-grade stainless steel). |
| Dimensions & Capacity (Standard DIN 9797) |  |
| * Nominal Volume | 200 L – 205 L. |
| * Nominal External Width | 670 mm – 680 mm. |
| * Nominal External Length | 700 mm – 710 mm. |
| * Nominal External Height | 700 mm – 715 mm. |
| Construction & Features |  |
| * Base Reinforcement | The base of the container must be additionally reinforced (e.g., by a 3 mm thick, tightly welded steel plate or similar industrial reinforcement) to ensure optimum stability and resistance against deformation. |
| * Handling | Designed with handle bars suitable for transport by standard lifting and pushing equipment. |
| * Wheels | Equipped with 4 heavy-duty, robust swivel wheels (min. diameter 150 mm) suitable for intensive industrial use. |
| * Drainage | Seamlessly welded interior with smooth surfaces and rounded corners (minimum R3 radius) for easy cleaning and compliance with HACCP hygiene requirements. |
| * Pouring Lip (Optional) | Containers must have a standard design without a pouring lip (skirtingą JAV USDA versiją) unless special request is made by the Procuring Authority. |
| Complete Functional Set (Mandatory) | The Supplier must ensure the delivery of ten fully compliant and functional Eurobins. The offer must explicitly include: |
|  | • Containers must be ready for immediate use. |
|  | • CE Declaration of Conformity for the product. |
| Guarantee | Minimum 24 months full manufacturer or supplier warranty against material or manufacturing defects. |