Annex No. 2

TECHNICAL SPECIFICATION

PURCHASE OF INDUSTRIAL TUNNEL WASHING MACHINE FOR GN CONTAINERS & BAKING SHEETS

place, date

The following table:

| Parameter | Requirement |
| --- | --- |
| THE PARTICIPANT shall provide information or proof against each requirement. |  |
| Quantity | 1 pcs. – Automatic conveyor/tunnel washing machine for containers, transport baskets, and baking sheets. |
| Purpose | Industrial washing and drying of large items such as GN 1/1 containers, baking trays (600x400 mm or similar), and transport baskets. The machine must be designed for heavy-duty use in a central kitchen or bakery environment. |
| Direction of Operation | Left to Right (or as per final layout confirmation). |
| Capacity / Performance | • Throughput: 200 – 250 GN 1/1 containers per hour (at minimum 3 minutes contact time according to DIN SPEC 10534 or equivalent standards for hygiene). • Transport Speed: Adjustable range 1.1 – 1.4 m/min. |
| Dimensions | • Total Length: 8300 mm – 8500 mm. • Passage Width: 730 mm – 750 mm. • Passage Height: 580 mm – 620 mm. • Loading Section Length: 750 mm – 900 mm. • Unloading Section Length: 1300 mm – 1500 mm. |
| Washing Zones | The machine must consist of the following functional zones: 1. Pre-wash zone: Heavy-duty removal of coarse soil. 2. Main wash zone: Intensive cleaning with detergent. 3. Pump rinse zone: Recirculated rinsing. 4. Fresh water final rinse zone: Sanitizing rinse with fresh water. 5. Drying zone: High-performance drying section with air blowing (preferably from below/bypass for containers). |
| Water Consumption | • Final Rinse: 220 – 250 litres/hour (providing high efficiency). • Tank Filling: 350 – 400 litres. |
| Energy & Heating | • Heating Source: Electric. • Total Connected Load: 59.0 kW – 63.0 kW . • Voltage: 400V / 3N PE / 50 Hz. |
| Construction & Hygiene | • Material: Stainless steel AISI 304 construction (frame, tanks, panels, spray arms). • Design: Double-wall insulated body for heat retention and noise reduction. • Conveyor: Specialized belt/transport finger system suitable for securely holding containers and baking sheets in an inclined position for optimal draining. |
| Ventilation & Heat Recovery | • Heat Recovery: Integrated exhaust air heat recovery system to pre-heat incoming cold water and reduce energy consumption. • Exhaust Air Volume: Total 1800 – 2200 m³/h. |
| Control System | • Interface: Electronic control panel with touch display or intuitive buttons. • Functions: Temperature monitoring, speed control, error diagnostics, automatic tank filling. |
| Complete Functional Set (Mandatory) | Supplier Responsibility: The supplier must ensure the delivery of a fully functional machine ready for the specific task of washing GN trays and baking sheets. The offer must include: • All necessary internal pumps and motors. • Dosing units for detergent and rinse aid. • Splash guards/curtains at entry and exit. • Emergency stop switches at both loading and unloading ends. • Heat recovery module. • Installation materials (feet, cables). • Commissioning and staff training. |
| Documentation | To be supplied in Latvian or English: user manual, electric diagram, maintenance instructions, CE Declaration of Conformity. |
| Compliance | Equipment must comply with EU Machinery Directive 2006/42/EC, DIN 10510/10534 standards (hygiene requirements). CE marking mandatory. |
| Guarantee | Minimum 24 months full manufacturer or supplier warranty. |

Name, surname, position

signature